## In summary (Table 4.2)

|  | Type | Instrument | Measurements | wavelengths | Derived products | Sampling |
| --- | --- | --- | --- | --- | --- | --- |
| Continuous measurements (coastal site) | Atmospheric measurements | CIMEL automatic photometer CE-318 | Sky radiances, and attenuation of the direct solar beam | 440, 670, 870, and 1020 nm, + 440 polarized | aerosol optical thickness (a()) and aerosol types | Variable, down to 10 min. |
| Continuous measurements (buoy) | Above surface measurements | Satlantic 200 | Downwelling irradiance | 412, 443, 490, 510, 560, 665, and 683 nm | Es() | 15 min (one minute at 6 Hz each 15 minutes from sunrise to sunset; “burst mode”) + one minute at 6 Hz each hour during night |
|  | Sub-surface measurements | Satlantic 200 | Downwelling irradiance at 4 meters | idem | Ed() |  |
|  |  | Satlantic 200 | Upwelling irradiance at 4 meters | Idem | Eu() |  |
|  |  | Satlantic 200 | Upwelling radiance at 4 meters | Idem | Lu() |  |
|  |  | Satlantic 200 | Downwelling irradiance at 9 meters | Idem | Ed() |  |
|  |  | Satlantic 200 | Upwelling irradiance at 9 meters | Idem | Eu() |  |
|  |  | Satlantic 200 | Upwelling radiance at 9 meters | Idem | Lu() |  |
|  |  | Chealsea’ MiniTracka fluorometer | Chlorophyll fluorescence at 5 meters | N/A | Proxy to Chl |  |
|  |  | Idem | Chlorophyll fluorescence at 9 meters | N/A | Idem |  |
|  |  | Wetlabs’ C-star transmissometer | Beam attenuation | 660nm | Attenuation coefficient, c() |  |
|  |  | Seabird’ SBE 37 SI | CTD | N/A | Salinity, depth, temperature |  |
|  |  | Hobilabs’ Hydroscat-II | backscattering | 442, 560 nm | Backscattering coefficient, bb() |  |
|  |  | EZ-compass III | Tilt sensor and compass | N/A | tilt and orientation of the buoy (w.r.t. the sun position) |  |

- Continued on the next page

| Monthly servicing to the buoy (systematic measurements; weather permitting and except when instrument would be unavailable) | Vertical profiles (0-150 m) | Satlantic SPMR | Downwelling irradiance + above surface irradiance | 380, 412, 443, 455, 490, 510, 530, 560, 620, 665, 683, 705, 780 nm | Spectral Ed() | N/A |
| --- | --- | --- | --- | --- | --- | --- |
|  | Idem | Satlantic SPMR | Upwelling irradiance | Idem | Spectral Eu() |  |
|  | Above the surface | Satlantic SPMR | Downwelling irradiance above the surface | Idem | Spectral Es() |  |
|  | Above-water measurement | SIMBADA | Water-leaving radiance + sky reflection + attenuation of the direct solar beam | 350, 380 , 412, 443, 490, 510, 565, 620, 670, 750 et 870 nm | Water-leaving radiance and aerosol optical thickness |  |
|  | 0-400 m | SeaBird | Classical CTD package | N/A | Conductivity, temperature, pressure, fluorescence, Oxygen |  |
|  | 0-400 m | AC-9 Installed on the rosette | Absorption and attenuation | 412, 443, 488, 510, 532, 555, 630, 676, 715 | Absorption and scattering coefficients, a(), c() (and b = c – a) |  |
|  | 0-400 m | Wetlabs ECO-VSF | Backsattering coefficient |  |  |  |
|  | 0-400 m | Wetlabs CDOM fluorometer | CDOM fluorescence | N/A | Qualitative description of CDOM |  |
|  | Discrete sampling for further analysis | 12 bottle rosette | HPLC | N/A | Phytoplankton pigments |  |
|  | Idem | Idem | Licor | N/A | Particles and phytoplankton absorption coefficient, ap(), a() |  |