Sea Surface fCO2 measurements in the Indian and Southern Oceans

obtained during MINERVE-29 associated to JGOFS/ANTARES-2 cruise

onboard the R.V. Marion-Durfresne (IPEV), 28/1-22/3/94

(Chief Scientist, M.Fiala, Lab. Arago, Perpignan, FRANCE)

## Prepared by N.Metzl and C.Brunet

Laboratoire de Biogeochimie et Chimie Marines (LBCM), UMR 7094

Institut Pierre Simon Laplace

Universite P. et M. Curie - Case 134

4, place Jussieu - 75252 PARIS Cedex 5 - FRANCE

http://www.lbcm.jussieu.fr/

### Method

The sea surface fugacity of CO2 (fCO2) was measured onboard the reasearch vessel Marion-Dufresne (IPEV) during the French JGOFS/ANTARES-2 by C. Brunet and C.Bossonet (LBCM, Paris) in the frame of the project MINERVE-29. The fCO2 measurements technique has been described for other MINERVE cruises conducted during years 1990-1995 in the Indian and Southern Ocean (Poisson et al.,1993; Metzl et al. 1995, 1999). This instrumentation was also used by our group during the international at-sea intercomparison of fCO2 systems conducted in 1996 in the North-Atlantic (Kortzinger et al., 2000).

In short, sea surface water is continuously equilibrated using a "thin film" type equilibrator thermostated with surface seawater. The CO2 in the dried gas is measured with a non-dispersive infrared analyser (NDIR, Siemens Ultramat 5F). Standard gases for calibration (269.9, 349.7, 489.6 ppm) and atmospheric CO2 are measured every 7 hours. To correct measurements to in situ data, we used polynomials given by Weiss and Price (1980) for vapour pressure and by Copin-Montégut (1988, 1989) for temperature. Depending the location (warm and cold waters) the temperature in the equilibrium cell was 0.50°C to 5°C warmer than the SST during MINERVE-29 cruise. The differences between temperature in the equilibrium cell and SST is much larger than during previous cruises (e.g. Poisson et al., 1993). This is explained by the sea surface water pumping system which was not operating in an homogeneous way durung this cruise. As long as the difference of temperature is taken into account in the fCO2 corrections, the sea surface fCO2 measurements are accurate. However, for short periods, the problems encountered with the seawater pumps may have change the equilibration time and thus any measurements of seawater properties (salinity, fCO2,…). At present, the possible off-sets have not been unidentified.

Based on different cruises analysis, the oceanic fCO2 data should be accurate to about  0.7 µatm. All parameters presented in this data-set correspond to the average of about 60 records made during 10 minutes.

The fCO2 data obtained during MINERVE cruise have been included in synthesis studies of air-sea CO2 fluxes at regional scale in the SubAntarctic zone (Metzl et al., 1999), for constructing global scale pCO2 climatologies (Takahashi et al., 2002) and for comparing and/or validating ocean models (e.g. Louanchi et al., 1996; Metzl et al., 1998).

# File description

The file MIN29CO2W.xls contains all the results of sea surface fCO2 measurements (and associated properties) made onboard during the cruise MINERVE-29. The columns of the file include: Date-hour (dd/mm/yy hh:mn), Latitude (degree.degree), Longitude (degree.degree), atmospheric pressure (mb), sea surface water fCO2 fugacity (µatm) normalized at 1atm, FCO2 (1013) and at local pressure, FCO2(patm), fluoresence (in relative units), temperature in the equilibrium cell (°C), sea surface temperature (°C), and sea surface salinity (PSU). The first date, first line of the data set, is 28/01/94 at 01:16.

For more information or if you have questions concerning these data, please contact N.Metzl (metzl@ccr.jussieu.fr) or C.Brunet (brunet@ccr.jussieu.fr)

References:

Copin-Montégut, C., 1988. A new formula for the effect of temperature on the partial pressure of CO2 in seawater. Mar. Chem., 25, 29-37.

Copin-Montégut, C., 1989. A new formula for the effect of temperature on the partial pressure of CO2 in seawater. Corrigendum. Mar. Chem., 27, 143-144.

Kortzinger,  A., L. Mintrop, D.W.R. Wallace, K.M. Johnson, C. Neill, B. Tilbrook, P. Towler, H.Y. Inoue, M. Ishii, G. Shaffer, R.F. Torres Saavedra, E. Ohtaki, E. Yamashita, A. Poisson, C. Brunet, B. Schauer, C. Goyet, G. Eischeid, The international at-sea intercomparison of fCO2 systems during the R/V Meteor Cruise 36/1 in the North Atlantic Ocean, Marine Chemistry, 2(2-4), 171-192., 2000.

Louanchi, F., N. Metzl, A. Poisson, 1996. Modelling the monthly sea surface fCO2 fields in the Indian Ocean. Marine Chemistry, 55, 265-279.

Metzl, N., A.Poisson, F. Louanchi, C. Brunet, B. Schauer , 1995.  Spatio-temporal distributions of air-sea fluxes of CO2 in the Indian and Antarctic Oceans: a first step. Tellus, 47B, 56-69.

Metzl, N., F.Louanchi and A.Poisson, 1998. Seasonal and interannual variations of sea surface carbon dioxide in the subtropical Indian Ocean. Marine Chem., 60, 131-146.

Metzl, N., B.Tilbrook, A.Poisson, 1999. The annual fCO2 cycle and the air-sea CO2 fluxes in the sub-Antarctic Ocean Tellus, 51B, 4, 849-861.

Poisson,  A., N. Metzl,  C. Brunet,  B. Schauer, B. Brès, D. Ruiz-Pino and F. Louanchi, 1993. Variability of sources and sinks of CO2 and in the Western Indian and Southern Oceans during the year 1991. J. Geophys. Res., 98, C12, 22759-22778.

Takahashi, T., S C. Sutherland, C. Sweeney, A.Poisson, N.Metzl, B.Tilbrook, N.Bates, R.Wanninkhof, R.A. Feely, C.Sabine and J.Olafsson and Y. Nojiri, 2002. Global Sea-Air CO2 Flux Based on Climatological Surface Ocean pCO2, and Seasonal Biological and Temperature Effect. Deep Sea Res. II, Vol 49, N 9-10, 1601-1622.

Weiss, R.F. and B.A. Price, 1980. Nitrous oxide solubility in water and seawater. Marine Chem., 8, 347-359.