ANTARES-4  METADATA

PROJECT TITLE: Ammonia concentrations in the surface waters of the Indian Ocean region of the Southern Ocean.

NAME OF PRINCIPAL INVESTIGATOR: Frank Dehairs

(Analyst: Ros Watson)

Contact information below is for Ros Watson

Address: CSIRO Marine Research, Castray Esplanade, Hobart, Tasmania, Australia, 7000

Phone number: +61 3 62 325269

Fax number: +61 3 62 325000

Email address: Ros.Watson@marine.csiro.au

BRIEF DESCRIPTION OF PROJECT

This investigation was carried out during the Antares-4 expedition in the region between 42-47°S latitude and 60-66°E longitude.  It forms part of the French Joint Global Ocean Flux (JGOFS) research. The project received funding from the Belgian JGOFS program. The aim is study the ammonia concentrations in surface waters across the subantarctic and subtropical fronts in the Indian Ocean.

TITLES OF ANTICIPATED PUBLICATIONS

It is expected that there will be a paper published that will describe the ammonia distribution in the study area.

(Will need to confere with Frank Dehairs regarding the content and any other possible publications.)

DESCRIPTION OF DATA

Ammonia/ammonium concentrations were measured in upper 300m of the water column. The method used was gas-diffusion FIA technique with fluorescence detection (Watson etal, in prep). This technique uses a gas-diffusion cell in a FIA system to diffuse ammonia from the seawater sample into the reagent stream. The reagents (ortho-phthaldialdehyde and sulphite) react with the ammonia to form a compound that can be detected by fluorescence. Analysis was done on board the Marion Dufresne on either freshly collected or frozen samples. With the exception of 2 CTD casts (one from the transect series and one from a CMF cast at site 8), all samples were measured in duplicate during the cruise.

Samples for analysis were collected from 58 CTD casts  during the study period.

These casts included:

- the core parameter casts from the process stations (sites 3, 7 and 8), sampled every 12 hours over a 4 day period.

- the CMF diurnal study casts from the process stations (sites 3, 7 and 8), sampled every 2 hours over a 24 hour period.

- the 7 CTD casts from the transect crossing the fronts (between sites 4 and 5).

- the shallow CTD casts from P and G sites within the TOWYO grid.

Although the laboratory analysis is complete, the data analysis has still to be completed. It is expected that this will be done within 6 months of the completion of the cruise.

The detection limit for the analysis in approximately 20nM. Replicate analysis is generally within 20nM or 10%, a better picture of the accuracy will be available after the completion of data analysis.

DATA DESCRIPTION

The data file name is ROSNH4.XLS.

Headings in the file are:

Date (UT) - UT Date of sampling.

Station - Station or site code.

# OPA - Operation number

# CTD - CTD cast code.

# Niskin - Code for the niskin bottle.

Ammonia - Ammonia concentration of the sample in nM.