## Contribution to cruise report Marion Dufresne 113 Antares 4

by R T Pollard, J F Read and B Ollivier

### Project title

The work described here is a contribution to the physical objectives of the Antares 4 cruise, namely: “Mesoscale dynamics of the 3-D frontal circulation in the Crozet Basin.”

### Scientific collaborators

The research is a collaboration between French and British physical oceanographers, from the Muséum National d’Histoire Naturelle led by Young-Hyang Park, and from the Southampton Oceanography Centre (Raymond Pollard and Jane Read). The SOC team (rtp@soc.soton.ac.uk, jfr@soc.soton.ac.uk) are at the Southampton Oceanography Centre, Empress Dock, Southampton SO14 3ZH, England.

Telephone: (44)1703-596433		Fax: (44)1703-596204

### Brief description of project

Previous surveys by the collaborators (Park et al., 1993; Read and Pollard, 1993) have shown that three major currents, the Agulhas Return Current, the Subtropical Front and the Subantarctic Front, combine or are in close proximity in the Southwest Indian Ocean sector of the Southern Ocean between about 40° and 65°E. Our objective was to survey these fronts in the centre of the Crozet Basin with mesoscale resolution in order to resolve their 3-D dynamics, their temporal variability and their effect on the biological and chemical distributions which are the central objective of SO JGOFS Project Antares 4. To resolve the mesoscale, and in particular to attempt to infer vertical circulations induced by ageostrophic dynamics, requires combined high resolution mapping of the current and density fields (e.g. Pollard and Regier, 1992). Similarly, it is possible to infer temporal changes in the surface geostrophic velocity field along an altimeter satellite track by “calibrating” the altimeter with current and density profiles along the satellite track as described by Challenor et al. (1996). Using a TOWYO winch to obtain CTD profiles to 1200m and the Marion Dufresne’s 750kHz ADCP to obtain current profiles to over 700m, we have the potential to describe mesoscale dynamics to a much greater depth than previously reported in the literature. Making use of the three 4-day long biochemical stations, we shall also examine the effect of tides on the current profiles, which Challenor et al. (1996) found to be a limiting factor in combining the current and geostrophic shear profiles.

### Responsibilities

While the physical data in this French JGOFS project are formally the responsibility of Y-H Park, it has been convenient to partition the detailed calibration, with CTD and TOWYO calibration being the responsibility of Park, and ADCP logging and calibration (including the navigation) being the responsibility of Pollard and Read. Thus details of navigation and ADCP processing are described here.

### Navigation

Navigation data were read in daily from the 1-second @GPS records logged on the Marion Dufresne’s central server, edited and averaged to one value every 30 seconds. The 30-second averages form the master navigation file. Editing required was as follows:

(a)	Dates were not monotonic. Daily, for about 15 minutes before 0000UT the day of month would increment by one prematurely and randomly. On 31 January the month incremented similarly. This problem was also found in TOWYO data interfaced to an independent GPS receiver (J-P Labat). Time within day remained correct, as did the positions. These forward day jumps were trapped and corrected.

(b)	The number of satellites would occasionally drop to zero, and the latitude and longitude would become constant. When the number of satellites recovered the positions would be poor for several data cycles. These data cycles were edited out prior to averaging.

(c)	On two occasions, the @GPS data stream was interrupted for an hour or two because of computer failures. However, GPS data being passed simultaneously to the ADCP were unaffected. The ADCP logger was set to average over 3-minute intervals and it was found that the 3-minute averaged position agreed closely (within 0.0001° = 10m) with the spot position at the midpoint of the 3-minute period. The ADCP 3-minute positions were therefore used to fill the gaps in the 30-second master file.

The accuracy of the positions was tested on two occasions when the ship was stationary, (a) docked at La ReUnion and (b) at anchor at Port aux Français, Kerguelen. On both occasions the fixes precessed in a manner typical of the degraded (non-military) gps positions. The standard deviations of the ensembles of fixes were 17m and 14m (longitude and latitude respectively) at La ReUnion, and 22m and 19m at Kerguelen. Later in the cruise, a gps-inertial system showed similar standard deviations of 24m. Because the downgraded positions precess over longer periods than 30 seconds, the accuracy of the 30-second averages should also be taken to be 20-30m in each of latitude and longitude.

### Winds

Data from the Gill sonic anemometer on the foremast were logged every 2 seconds. These data were converted into east and north components of wind relative to the ship using the known 69° difference between the anemometer zero heading and the ship’s head, and the ship velocity was vector added to give true winds. A 10-minute averaged file of true winds is available.

### ADCP logging and calibration

The 75kHz Acoustic Doppler Current Profiler was used throughout the cruise to obtain currents in the upper 700m of the water column. After changing the heading reference from gyro to Sercel 3-D GPS, calibrating the scaling factor and misalignment angle, and adding the velocity of the ship to the velocity of the water relative to the ship (which is what the ADCP measures), it is estimated that biases in the currents have been reduced to about 1-2 cm/s. The current profiles, with 8m vertical resolution and averaged over 1 hour, show bin to bin noise of 2-5 cm/s which can be reduced by further averaging. Thus accurate current profiles usually down to 700m are available for the whole cruise. Details are as follows.

#### Data logging and transfer

After testing various instrument setups at the start of the cruise, the following parameters were used throughout the main scientific programme: depth of transducer = 7m; blank beyond transmit = 4m; pulse length 8m; 90 x 8m bins. Thus midpoints of bins are at depths of 15 (8) 727m. Averaging interval = 3 minutes The DAS software supplied with the RDI 75kHz ADCP creates “pingdata.nnn” files (where nnn is an integer starting from 000), which were transferred daily to the central server, then read into PSTAR, which is the SOC (Southampton Oceanography Centre) data format and Fortran software package used for all processing described here. The data were appended into files corresponding to convenient sections of the scientific programme (Table 1).

#### Conversion from gyro to gps heading

The ADCP reads the gyro on every ping in order to convert velocity components fore-aft (avant-arriere) and port-starboard (babord-tribord) into east and north components. About 56-59 pings are averaged per 3-minute ensemble. However, while the gyro is the most reliable source of heading information, it has temporal drifts and heading dependent errors that result in significant errors in calculated currents. The first processing step is therefore to change the heading reference from gyro to 3D GPS. The Sercel system on Marion Dufresne provided heading from 3 GPS receivers from 22 January, just at the start of the main CTD and Towyo survey. Every 0.6 seconds, gps and gyro headings were simultaneously sampled and logged in @ATT2,1 ascii records, along with pitch, roll and pdop data. Because gps heading is affected by large pitch, roll and pdop, all data cycles with pitch or roll greater than ±5° or pdop greater than 5 were discarded, also all data cycles for which g = (gps heading - gyro heading) was less than -5° or greater than 0°, as the peak of the histogram of g was at -2.7°. Remaining data cycles were averaged over 2 minute periods to remove wave periodicities. Data gaps were interpolated, making use of the information in Figs 1 and 2 if ship heading changed during a gap.

Fig. 1 shows the scatter of g values as a function of gyro heading. Fig. 2 shows g as a function of time. The variations of g with heading reflect the fact that gyro compasses have errors which are a function of heading. The variations of g with time show both short and long term changes in g. The short term variations, most apparent at the start and end of the full depth CTD casts on 22-24 January, are the result of 84 minute period Schuler oscillations, a natural period of oscillation of gyrocompasses. The long term changes seen in Fig 2 appear to be the result of two jumps in gyro calibration, which result in three clusters of points at (for example) headings just less than 360° (Fig. 1).

The average offset of 2.7° between the gyro and gps headings was removed, as it is assumed that the gyro is well calibrated in the mean, whereas the Sercel installation could be in error, because of lack of time to calibrate it.

Finally, the averaged g values were merged onto the ADCP 3-minute ensembles, and the velocity vector of the water relative to the ship was rotated by g so that all velocities are referenced to the unvarying gps reference heading instead of the varying gyro zero.

#### Calibration of misalignment angle and scaling factor

Calibration of the ADCP installation misalignment angle phi (relative to the gps zero heading) and the ADCP scaling factor A was carried out using the procedure described by (Pollard and Read, 1989). Water track calibration was performed on two occasions. On 22/23 January, 7 full depth CTD stations (4023-4029) were occupied. Assuming the current was the same for 15 minutes of passage just prior to a station and the first 15 minutes on station, and similarly at the end of each station, 13 calibrations were obtained. On 6 February, the ship steamed along a Topex-Poseidon track then returned along it to the position chosen for long station 7. Assuming the current variation with latitude was the same on the outward leg and the return leg, a further 13 calibration values were obtained. Statistics for the two sets of calibrations were

Mean phi	std devn of	mean A	std devn of

(degrees)	mean phi		mean A

13 CTD comparisons		0.30	0.11	0.9734	0.0042

13 reverse track estimates	0.28	0.10	0.9814	0.0031

Calibration values chosen	0.30	(±0.10)	0.978	(±0.004)

These values were applied to all vectors of water velocity relative to the ship.

At the end of the cruise, two runs for bottom track calibration were completed near Crozet. The values of (A,phi) calculated from these runs were (0.9756, 0.83°) and (0.9627, 0.82°) for 21 minute runs. Why these differ from the earlier calibration is not clear at the time of writing. However, even if the calibrations used are in error by 0.008 (A) and 0.5° (phi), the error in absolute velocities during the 4 knot TOWYO runs would be less than 2 cm/s.

#### Deriving absolute currents from navigation

The 3-minute ADCP ensembles are recorded with a time stamp of the end of the averaging interval. This enabled us to merge in the gps latitude and longitude at the end of each interval, and hence to calculate the velocity of the ship over exactly the same interval as the ADCP average. Thus absolute water velocities are calculated for each 3-minute interval. However, the error in the ship velocity vector is considerable over such a short interval (around 20m x 2 ∏ 180sec = 15 cm/s) so the data are further averaged over 15 minutes.

#### Accuracy of derived currents

The errors in water track velocities are proportional to the ship speed S. Errors in phi result in bias errors of order S x sin(phi) = 1.4 cm/s for S = 16 knots (8 m/s) and error in phi of 0.1° (see table above). Errors in A result in bias errors of order S x A = 3.2 cm/s for the same S (8 m/s) and error in A of 0.004. Errors in gps position determination result in random errors of order

Error in position x 2 ∏ time interval = 3-4 cm/s

for 15 minute averaging. Remaining errors are random errors inherent in the ADCP sampling, and depend on the number of pings per ensemble, percentage of good pings per ensemble, pulse length, bin size, noise in the water, etc. Based on the noise apparent on typical profiles with 8m bins and high percent good readings, the random noise after 1 hour averaging is about 5-10 cm/s. This noise can be reduced by further temporal or depth averaging.

### References

Challenor P., R. T. Pollard, J. F. Read and R. Tokmakian (1996) Measuring surface currents in Drake Passage from altimetry and hydrography. Journal of Physical Oceanography, 26, 2748-2759.

Park Y.-H., L. Gamberoni and E. Charriaud (1993) Frontal structure, water masses and circulation in the Crozet Basin. Journal of Geophysical Research, 98, 12361-12385.

Pollard R. T. and J. Read (1989) A method of calibrating shipmounted acoustic doppler profilers and the limitations of gyro compasses. Journal of Atmospheric and Oceanic Technology, 6, 859-865.

Pollard R. T. and L. A. Regier (1992) Vorticity and vertical circulation at an ocean front. Journal of Physical Oceanography, 22, 609-625.

Read J. F. and R. T. Pollard (1993) Structure and transport of the Antarctic Circumpolar Current and Agulhas Return Current at 40°E. Journal of Geophysical Research, 98, 12281-12295.

## Table 1 ADCP files

File Name	start date	time	end date	time	comments

(UT)		(UT)

adpmd102	990107	2257	990109	0340	initial setup 120s samp. interval

adpmd103	990109	0343	990110	1220	LaR - Amsterdam - St Paul

adpmd104	990110	1818	990113	1520	St Paul - Point 1 - Point 1’

adpmd105	990113	1535	990115	0314	Point 1’ - Kerguelen

adpmd106	990115	1113	990116	0531	Kerguelen - Point 1’

adpmd107	990116	0616	990117	2213	Point 1’ - 2 - start of long stn 3

adpmd108	990117	2216	990122	0702	Time series at long station 3

adpmd1ty	990122	0701	990205	1200	Whole of towyo survey

adpmd118	990205	1203	990210	2023	T/P line + time series at stn 7

adpmd119	990210	2026	990216	0539	OPCT line + time series at stn 8

adpmd120	990216	0542	990218	0300	passage stn 8 to Crozet

adpmd121	990218	1400	990221	0043	passage from Crozet towards La ReUnion