# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name (list of the measured parameters):

- Spectral irradiance above the sea surface modelled with SBDART: Ed0

File: IC2015_SBDART_AllDays.zip

- Daily metrics for PAR just above the sea-surface for both SBDART output and in situ measurements.

File for SBDART output metrics: PAR0_sbdart_dailyMetrics.GE2015.IC.xlsx

File for in situ sensor metrics: PAR0_insitu_dailyMetrics.GE2015.IC.xlsx

Variables defined as follows for both SBDART and in situ data:

Ship operations log

1. yearUTC

2. monthUTC

3. dayUTC

4. doyUTC: day of year.

5. timeUTC: decimal time.

6. lat: Latitude N.

7. lon: Longitude E.

PAR data

All variables starting with "PAR" have units of µE m-2 s-1. They correspond to a single value or to the mean during a given period. In the latter case, they are followed by a variable with the same name except that PAR has been replaced by N.

8. PARsam: measurement closest to the time of sampling, irrespective of the frequency of PAR measurements.

9. PARsam_interp: PAR linearly interpolated between the two closest measurements, irrespective of the frequency of PAR measurements.

10. PARp1h: mean PAR during the 1-h period prior to sampling.

11. Np1h: corresponding data counts (note this variable may not differ from PARsam if the measurement frequency is ≥1h).

12. PARp3h: mean PAR during the 3-h period prior to sampling.

13. Np3h: corresponding data counts (note this variable may not differ from PARsam if the measurement frequency is ≥1h).

14. PARp24h: mean PAR during the 24-h period prior to sampling.

15. Np24h: corresponding data counts.

16. PARp48h: mean PAR during the 48-h period prior to sampling.

17. Np48h: corresponding data counts.

18. PARdayLOC: mean PAR during the local day (ie 24h starting at 00:00 local time).

19. NdayLOC: corresponding data counts.

20. PARnoon1hLOC: mean PAR during the 1-h period centered around the local day noon.

21. Nnoon1hLOC: corresponding data counts.

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME:   GreenEdge – ICE CAMP – 2016

Date begin: April 2016 (day of year 91 earliest date for SBDART output)

Date end: Mid-August 2016 (day of year 228 latest date for SBDART output)

Chief Scientist: Marcel Babin

Address:

Université Laval - UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

OPERATION (if Relevant)

Sampling method:

Station number-Cast number:

Operation code:

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # BABIN Marcel | # Takuvik | # +1 418 656 2131 #5230 | # +1 418 656 2339 | # marcel.babin@takuvik.ulaval.ca |

### DATASET contact

idem

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # GALÍ Martí | # Barcelona Supercomputing Center |  |  | # marti.galitapias@bsc.es # marti.gali.tapias@gmail.com |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):  Baffin Bay

Location:

LATITUDE: 66 to 70 degres North

LONGITUDE: 55 to 65 degres West

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

For details on in situ measurements, see corresponding reports by Brent Else and Tonya Burgers.

Instrument Type: Radiometer

Manufacturer: Kipp & Zonen

Model: PAR Lite

Instrument Features / Calibration:

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling:

Analytical procedure: (briefly, could be a short recall to a published reference):

Methods described in section 2 of the following document: http://www.obs-vlfr.fr/proof/ftpv/greenedge/db/DATA/COPS/DATA/GE2016.AMIS.AMOW/GE.Optics.AOP.report.JAN.2018.html

SBDART model described by Richiazzi et al. (1998), implemented following Laliberté et al. (2016).

Units:

Spectral irradiance Ed0 (SBDART output): [µmol photons m-2 s-1 nm-1]

PAR0 Daily Metrics: [µmol photons m-2 s-1]

Sensor Precision:

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery:

### REFERENCES BIBLIOGRAPHIQUES

Ricchiazzi, P., Yang, S., Gautier, C., & Sowle, D. (1998). SBDART: A research and teaching software tool for plane-parallel radiative transfer in the Earth's atmosphere. Bulletin of the American Meteorological Society, 79(10), 2101-2114.

Laliberté, J., Bélanger, S., & Frouin, R. (2016). Evaluation of satellite-based algorithms to estimate photosynthetically available radiation (PAR) reaching the ocean surface at high northern latitudes. Remote Sensing of Environment, 184, 199-211.