Report on the utilization of the MVP200 during the OUTPACE cruise

(February/April 2015)

Lucio Bellomo, Gilles Rougier

1. Long-Duration Stations

LD-A (lon: 164.688, lat:-19.213) - 24/02 @ 11h UTC → 03/03 @ 11h UTC

We have performed:

| Operation | Description | Sensors | Comments |
| --- | --- | --- | --- |
| Cross | Regular cross, 3 transects, 40 km x 40 km square | CTD*,** + fluo + LOPC | - Unexploitable Salinity (S) - Attention to the quality of Temperature (T) measurements, due to the partially “hidden” position of the sensor |
| MVP_before | Slightly smaller zigzag, 7 transects instead of 8, 20 km x <20 km square | CTD** + fluo + LOPC | Same as for Cross |
| MVP_after | Regular zigzag, 8 transects, 20 km x 20 km | SVPT*** + fluo + LOPC | Unexploitable T and S |

*     Broken Conductivity sensor (Fig. 1)

**   Placed in the back of the fish (Fig. 2 shows the SVPT, but the position of the CTD is the same) so that
       the T sensor is  only partially in the free flow of water (it is “shaded” by the metallic structure of the
       fish)

*** Placed in the back of the fish (Fig. 2), where T and D sensors are completely not in the free flow of
       water

Fig. 1: AML microCTD with the position of the Conductivity (C) and Temperature (T) sensors

Fig. 2: Initial location of the Valeport SVPT, giving very bad measurements due to the "hidden" position of the Pressure (P) and Temperature (T) sensors

LD-B (lon: 189.142, lat: -18.24) - 12/03 @ 10h UTC → 21/03 @ 09h UTC

We have first changed the position of the Valeport SVPT. It has been put, thanks to a metallic tool built by the mariners onboard, in the middle of the fish, outside the protective plastic “wings”, to have it in the water flow without obstacles  (Fig. 4). We have then performed:

| Operation | Description | Sensors | Comments |
| --- | --- | --- | --- |
| Cross | Not at all a cross! 3 long transects (Fig. 3) | SVPT* + fluo + LOPC | - Acceptable S, but still very noisy and a little underestimated wrt the Rosette's CTD |
| MVP_before | Slightly smaller zigzag, 7 transects instead of 8, 20 km x <20 km square | SVPT* + fluo + LOPC | Same as for Cross |
| MVP_after | Just 1 long transect in-out of the chrlophyll-a patch | SVPT* + fluo + LOPC | Same as for Cross |

* Placed in the middle of the fish, our new position, where measurements should be clean as the sensors are
   well in the water flow (see Fig. 4)

Fig. 3: Average Chlorophyll-a concentration (CLS data) with MVP track. (Black) Cross, (gray) MVP_before, (white) MVP_after

Fig. 4: New "home-made" position of the Valeport SVPT in the middle of the fish, fully in the water flow.

LD-C (lon: 194.06, lat: -18.42) – 22/03 @ 22h UTC → 29/03 @ 09h UTC

Only the Cross has been performed here. Almost at the end of it (we were lucky), the MVP stopped working because of a thick bend along the cable (see Fig. 5) that prevented the downcast to be performed until the end (Max Cable Out or Max Depth). There was no time for the MVP_before anyway, but the MVP_after was replaced by an “old-style”, almost North-South, CTD transect (CTD_after).

| Operation | Description | Sensors | Comments |
| --- | --- | --- | --- |
| Cross | Regular cross, 3 transects, 40 km x 40 km square | SVPT* + fluo + LOPC | - Acceptable S, but still very noisy and a little underestimated wrt the Rosette's CTD |
| CTD_after | Just 1 long transect with 7 classic Rosette CTD's | All Rosette's sensors | CTD 200, 201, …, 206 |

* Placed in the middle of the fish, our new position, where measurements should be clean as the sensors are
   well in the water flow (see Fig. 4)

Fig. 5: Cable bending causing the MVP to interrupt its downcasts

2. Post-calibration tests

| Test # | MVP file(s) | MVP configuration | MVP comments | CTD file(s) | CTD comment | Other instr's file(s) | Other comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2015-02-25 _111401 | AML uCTD with broken C | Last zigzag profile (MVP_before) | out_c_021 | Made right after the MVP |  |  |
| 2 | 2015-02-26 _094058 | AML uCTD with broken C | Station mode, vertVel = 1 m/s | out_c_030 | Made right after the MVP |  |  |
| 3 | 2015-02-27 _115523 _121226 _123429 | AML uCTD with broken C | Station mode, 1 < vertVel < 2 m/s | out_c_037 | Made right after the MVP | RBRprofile_3 | RBR attached to the back of the fish |
| 4 | 2015-03-02 _080328 _081950 _084304 | SVPT, bad position (rear of the fish) | Station mode, vertVel = 1 m/s | out_c_063 | Made right before the MVP |  |  |
| 5 | 2015-03-06 _230819 _231503 _233357 _234039 _234729 _235418 2015-03-07 _000114 | SVPT, “home-made” position | Zigzag mode, made on purpose | out_c_080 | Made right after the MVP |  |  |
| 6 | 2015-03-08 _061050 | SVPT, “home-made” position | Station mode, vertVel = 1 m/s | out_c_087 | Made right before the MVP | VMP_profile_6 | VMP profile made right after the MVP |
| 7 | 2015-03-14 _225233 2015-03-15 _003131 | SVPT, “home-made” position | During zigzag (MVP_before) | out_c_104 | Made at one of the edges of MVP_before |  |  |