# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

UQAR-IOPs-package

The data set includes the following parameters:

- Conductivity-Temperature-Depth (CTD) profiles

- Spectral backscattering coefficients at 6 wavelengths (394, 420,470, 532, 620, 700 nm)

- Spectral total absorption coefficients at  405 nm from 360 to 764 nm

- CDOM fluorescence (ex/em:370/420, 370/465; 370/500, 394/)

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   GreenEdge – AMUNDSEN – 2016	LEG : 1

Date begin : June 9 2016

Date end :  July 17 2016

Chief Scientist: Marcel Babin (Leg 1A) ; Jean-Eric Tremblay (Leg 1B)

Address :

Université Laval - UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

OPERATION (if Relevant)

Sampling method : The Optical package frame was deployed at 28 stations from the Amundsen bow using the A-frame.

Station number-Cast number :

G100, G102, G104, G107, G113, G201, G204, G207, G300, G308, G312, G318, G324, G403, G409, G413, G418, G507, G512, G519, G603, G600, G605, G615, G604.5, G703, G707, G713, G719

Operation code :  IOP

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Simon Bélanger | # 300 Allées des Ursulines, Rimouski, QC G5L 3A1 | # 418-723-1986 poste 1969 |  | # simon_belanger@uqar.ca |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Simon Bélanger | # 300 Allées des Ursulines, Rimouski, QC G5L 3A1 | # 418-723-1986 poste 1969 |  | # simon_belanger@uqar.ca |

| Date | Station | Station Type | Lat | Lon | Deployement |
| --- | --- | --- | --- | --- | --- |
| Leg 1a |  |  |  |  |  |
| 2016-06-09 | G100 | FULL | 68N28.92 | 56W47.06 | Amundsen |
| 2016-06-10 | G102 | FULL | 68N29.78 | 57W28.93 | Amundsen |
| 2016-06-11 | G104 | BASIC | 68.501 | -58.146 | Amundsen |
| 2016-06-11 | G107 | FULL | 68N30.4 | 59W16.7 | Amundsen |
| 2016-06-13 | G113 | BASIC | 68N26.95 | 61W22.33 | Amundsen |
| 2016-06-14 | G201 | BASIC | 68N37.55 | 59W56.67 | Amundsen |
| 2016-06-15 | G204 | FULL | 68.7053 | -59.2532 | Amundsen |
| 2016-06-16 | G207 | FULL | 68N48.24 | 58W31.24 | Amundsen |
| 2016-06-17 | G300 | FULL | 68.999 | -56.7682 | Amundsen |
| 2016-06-18 | G308 | FULL | 69.0011 | -58.7379 | Amundsen |
| 2016-06-19 | G312 | BASIC | 69N0.83 | 59W36.23 | Amundsen |
| 2016-06-20 | G318 | BASIC | 68N59.51 | 60W58.77 | Amundsen |
| 2016-06-21 | G324 | BASIC | 68N58.02 | 62W20.182 | Amundsen |
| Leg 1b |  |  |  |  |  |
| 2016-06-25 | G403 | FULL | 68N 04.422 | 61W36.50 | Amundsen |
| 2016-06-26 | G409 | FULL | 68N05.824 | 60W00.63 | Amundsen |
| 2016-06-28 | G413 | BASIC | 68N6.9 | 58W56 | Amundsen |
| 2016-06-28 | G418 | BASIC | 68N6.888 | 57W45.2 | Amundsen |
| 2016-06-30 | G507 | FULL | 70 | -59.125 | Amundsen |
| 2016-07-01 | G512 | FULL | 69N59.521 | 60W19 | Amundsen |
| 2016-07-02 | G519 | FULL | 70N00.26 | 62W25.37 | Amundsen |
| 2016-07-03 | G600 | FULL | 69N59.83 | 63W1.07 | Amundsen |
| 2016-07-04 | G605 | FULL | 70N28 | 62W28 | Amundsen |
| 2016-07-05 | G615 | FULL | 70N30.3 | 59W32.1 | Amundsen |
| 2016-07-06 | G604.5 | FULL | 70N29.9 | 62W37.6 | Amundsen |
| 2016-07-07 | G703 | FULL | 69N30.1 | 58W43 | Amundsen |
| 2016-07-08 | G707 | FULL | 69N31.0 | 59W47.58 | Amundsen |
| 2016-07-09 | G713 | FULL | 69N30.0 | 61W34.8 | Amundsen |
| 2016-07-10 | G719 | FULL | 69N29.6 | 63W12.1 | Amundsen |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

| Instrument Type: | Manufacturer: | Model: | Instrument Features / Calibration: |
| --- | --- | --- | --- |
| CTD | Seabird | SBE-19 | 2009 |
| Backscattering meter | Hobilabs | Hydroscat-6 | March 2016 |
| Absorption meter | Hobilabs | A-sphere | March 2016 |
| CDOM fluorescence meter | WetLabs | ECO triplet | NA |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

Sampling:   Vertical profiles

Analytical procedure : (briefly, could be a short recall to a published reference):

The main parameters of this data set are the spectral backscattering coefficient (bb) and the spectral absorption coefficient excluding pure water (a). Both parameters required various corrections.  The data processing is ongoing.

Briefly, The total absorption from the A-Sphere will be corrected for temperature and salinity following Rottgers et al (2014) using the temperature and salinity data from the CTD. Backscattering coefficient from the Hydroscat-6 will processed following Maffione and Dana (1997) protocol. The light attenuation correction for bb will be performed following the method described in Doxaran et al (2016) using the corrected absorption coefficient from the A-Sphere. Particle backscattering (bbp) will calculated removing the pure water contribution calculated using the method of Zhang et al. (2009).

Units: bb and a are in m-1. Fluoresce are in raw counts.

Sensor Precision:

Estimated Date of Delivery : Winter 2017

### REFERENCES BIBLIOGRAPHIQUES

Doxaran, D., E. Leymarie, B. Nechad, A. Dogliotti, K. Ruddick, P. Gernez, and E. Knaeps (2016), Improved correction methods for field measurements of particulate light backscattering in turbid waters, Optics Express, 24(4), 3615–3637, doi:10.1364/OE.24.003615.

Maffione, R., and D. Dana (1997), Instruments and methods for measuring the backward-scattering coefficient of ocean waters, Applied optics, 36(24), 6057–6067.

Röttgers, R., D. McKee, and C. Utschig (2014), Temperature and salinity correction coefficients for light absorption by water in the visible to infrared spectral region, Optics Express, 22(21), 25093–25108, doi:10.1364/OE.22.025093.

Zhang, X., L. Hu, and M.-X. He (2009), Scattering by pure seawater: Effect of salinity, Optics Express, 17(7), 5698–5710, doi:10.1364/OE.17.012685.