# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Meteorological Tower measurements

- Air Temperature

- Relative Humidity

- Wind Speed and direction (relative and true wind)

- Atmospheric Pressure

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   GreenEdge – AMUNDSEN – 2016	LEG : 1a & 1b

Date begin : June 3, 2016

Date end :  July 14, 2016

Chief Scientist: Marcel Babin (Leg 1a) and Jean-Eric Tremblay (Leg 1b)

Address :

Université Laval - UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

OPERATION (if Relevant)

Sampling method : Various sensors mounted on meteorological tower located on the foredeck of the Amundsen. Campbell Scientific HMP45C212 temperature/relative humidity sensor,  RM Young 05106-10 wind monitor, and a Vaisala pressure transducer. Measurements are taken once a minute.

Station number-Cast number :  N/A (Sampled everywhere along cruise track)

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Brent Else | 2500 University Dr NW, Calgary, AB, # T2N 1N4 | # 1-403-220-2484 |  | # belse@ucalgary.ca |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Tonya Burgers | # 125 Dysart Road, Winnipeg, MB, Canada, R3T 2N2 |  |  | # tonya.burgers@umanitoba.ca |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: Baffin Bay

LATITUDE: 67 - 71 N

LONGITUDE  55 - 65 W

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type: Temperature/ Relative humidity sensor

Manufacturer: Campbell Scientific

Model: HMP45C212

Instrument Features / Calibration: Connects to Campbell Sci Data logger at base of tower, which records measurements once per minute

Instrument Type: Wind Monitor

Manufacturer: RM Young

Model: 05106-10

Instrument Features / Calibration: Connects to Campbell Sci Data logger at base of tower, which records measurements once per minute

Instrument Type: Pressure Transducer

Manufacturer: Vaisala

Model:

Instrument Features / Calibration: Connects to Campbell Sci Data logger at base of tower, which records measurements once per minute

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Sensors are mounted on a meteorological tower located on the foredeck of the Amundsen, and measurements are collected and saved on a data logger once per minute.

Analytical procedure : (briefly, could be a short recall to a published reference): N/A

Units:

Temperature: °C (+- 0.226 - 0.0028 x reading at -60 - 20 C)

Relative Humidity: % (+- 1.0 + 0.008 x reading at -20 - 40 C)

Wind Speed : m s-1 (for both relative and true wind speed) (+- 0.3 m/s)

Wind Direction: deg N (+- 3 deg). Azimuth angle (for relative wind speed an azimuth angle of 0° is from the bow and 180° is from the stern, for true wind 0° is north and 180° is south)

Atmospheric Pressure : kPa (+- 0.02 at 25C)

Sensor Precision:

Sensor accuracies are written in parentheses next to the units listed above.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery :  Data has been post-processed. We remove wind speed and directions coming from the stern of the ship (between 90° and 270°) as the body of the ship in this area distorts the wind field giving us false wind speed measurements. Data is ready for submission.

### REFERENCES BIBLIOGRAPHIQUES