# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name (list of the measured parameters): 
Total particulate Carbon concentration (TPC)

Total particulate Nitrogen concentration (TPN)

Suspended particulate matter concentration (SPM)

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   	GreenEdge – Amundsen	LEG : 1

Date begin : June 3rd, 2016

Date end : July 14th, 2016

Chief Scientist: Marcel Babin / Jean-Eric Tremblay

Address :

Université Laval

UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

### OPERATION (if Relevant)

Sampling method : Rosette Niskin bottles water samples and some ice cores

Station number-Cast number :  All Full Stations

Operation code : na

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Marcel Babin | # 1045 ave de la Médecine, Université Laval, Québec, QC, G1V0A6, Canada | # 418-656-2205 | # 418-656-2339/ | # marcel.babin@takuvik.ulaval.ca |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Flavienne Bruyant | # 1045 ave de la Médecine, Université Laval, Québec, QC, G1V0A6, Canada | # 418-656-2131 #5230 | # 418-656-2339 | # Flavienne.bruyant@takuvik.ulaval.ca |
| # Jade Larivière |  | # 418-656-2131 | # 418-656-2339 | # jade.lariviere@takuvik.ulaval.ca |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: Baffin Bay

LATITUDE:  65 – 75N

LONGITUDE 55 – 65W

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

CHN

Instrument Type:	CHNS/O elemental analyzer, gas chromatography, combustion/reduction method

Manufacturer: 	Perkin Elmer

Model: 2400 series II

Instrument Features / Calibration: Accurately weighed samples of a known standard, usually acetanilide (C8H9NO; Carbone, 71,09%; Azote, 10,36%), are first analyzed to calibrate the analyzer in milligrams of carbon and nitrogen.

SPM

Instrument Type:	Balance

Manufacturer: 	Mettler Toledo

Model: MX5

Instrument Features / Calibration: Calibrated weight have been used to validate the values.

Filter Weighing Kit (Mettler toledio #11122136) is used to weight the filter.

Capacity: 5.1g, Readability: 0.001mg

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: The seawater samples were collected from Niskin bottles on the rosette @ten depths per profile.

(1) The pre-ashed (450 C for 24 h) GF/F filters (25mm diameter) are weighted at home laboratory (Laval university);

(2) Seawater or ice samples are filtered onto pre-combusted 25mm Whatman GF/F then stored in an acid-washed petri dish;

 The same filter is used for both measurements;

(3) Filters are dried at 60˚C overnight;

(4) Petri dishes are vacuum seal in plastic until later analysis at Laval university;

(5) Filters are dried again for 24hours at 60˚Celcius;

(6) Filters carrying biomass are re-Weighted   SPM;

(7) Filters is then folded and encapsulated for CHN analysis;

(8) Capsules are being analyzed following the manufacturer CHN standard protocol.

Analytical procedure: (briefly, could be a short recall to a published reference):

CHN

TPC and TNC were determined on duplicate samples.

Field blank filters were periodically collected to substract data.

The analytical procedure is provided with the instrument User guide.

Units: ug L-1

Sensor Precision:

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : January 2017

### REFERENCES BIBLIOGRAPHIQUES