# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name (list of the measured parameters):

- Extracted Chlorophyll concentration [Chl a]

- Extracted Phaeopigments concentration [Phaeo]

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   GreenEdge – AMUNDSEN – 2016	LEG :  1

Date begin : June 3rd 2016

Date end :  July 14th  2016

Chief Scientist: Marcel Babin / Jean-Eric Tremblay

Address :

Université Laval - UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

OPERATION (if Relevant)

Sampling method :

Station number-Cast number :

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # BRUYANT Flavienne | # Takuvik | # +1 418 656 2131 #5230 | # +1 418 656 2339 | # flavienne.bruyant@takuvik.ulaval.ca |

### DATASET contact

idem

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):  Baffin Bay

Location:

LATITUDE:  55 to 65 degres West

LONGITUDE   66 to 70 degres North

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type: Fluorometer

Manufacturer:  Turner Designs

Model: 10-AU

Instrument Features / Calibration:

Acid ratio = 1.9416

K = 0.2636

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: -  samples collected from Niskin bottles at 10 depths

-  samples collected from melted ice cores

Analytical procedure : (briefly, could be a short recall to a published reference):

Seawater samples (with volumes ranging from 0.2 to 1 liter) were filtered on Whatman GF/F filters. Immediately after filtration, filters were placed into 10mL of frozen 90% acetone and placed in a freezer (-20) for a minimum of 24 hours. After extraction time, vials containing the filters were taken out of freezer in order to equilibrate with room temperature. Chlorophyll a fluorescence was measured before and after the addition of 3 drops of 5% HCl. The method used is based on those developed by Yentsch and Menzel (1963), Holm-Hansen et al. (1965) and Lorenzen (1967).  Note that concentrations of 'phaeopigments' are not a good measure of Chl a degradation products present in the sample since Chl b present in the sample will be measured as 'phaeopigments'.

Units: µg/L

Sensor Precision:

0.1%

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Quality-controlled HPLC data are usefull for cross-validation.

Estimated Date of Delivery :

end of January 2017

### REFERENCES BIBLIOGRAPHIQUES

Yentsch, C.S. and D.W. Menzel (1963). A method for the determination of phytoplankton chlorophyll and phaeophytin by fluorescence. Deep-Sea Res., 10 :221-23.

Holm-Hansen O., C.J.  Lorenzen, R.W. Holmes and J.D.H. Strickland (1965) Fluorometric determination of chlorophyll. J. Conseil, Conseil Perm. Intern. Exploration Mer, 30 : 3-15.

Lorenzen C.J. (1967) Determination of chlorophyll and Pheo-pigments : Spectrophotometric equations. Limnol. Ocean. Notes. 12 (2): 343-347