# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name (list of the measured parameters):

Inorganic nutrients (NO3, NO2, PO4, Si(OH)4)

PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   GreenEdge – AMUNDSEN – 2016	LEG :  1

Date begin : 3th June

Date end :  14th July 2016

Chief Scientist: Marcel Babin / Jean-Eric Tremblay

Address :

Université Laval - UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

OPERATION (if Relevant)

Sampling method :  Water sampling at the rosette

Station number-Cast number :  All stations except CTD stations (see attached file)

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Jean-Éric # Tremblay | # Laval # University, # bureau 2056-A | # (418) 656-2131 # poste 5340 |  | # Jean-Eric.Tremblay@bio.ulaval.ca |

| DATASET contact # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Gabrièle # Deslongchamps | # Laval # University, # bureau 2078 | # (418) 998-9218 |  | # gabriele.deslongchamps.1@ulaval.ca |
| # Jonathan # Gagnon | # Laval # University, # bureau 2230 | # (418) 656-2131 # poste 4511 |  | # Jonathan.Gagnon@qo.ulaval.ca |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: Baffin Bay

LATITUDE

LONGITUDE

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type: Autoanalyzer

Manufacturer:  Bran and Luebbe

Model: III

Instrument Features / Calibration: NA

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling

Samples for inorganic nutrients (nitrite, nitrate, orthophosphate and orthosilicic acid) were taken at all NUTRIENTS, BASIC and FULL stations to establish detailed vertical profiles. At the rosette, samples were pre-filtered through a GF/F filter and stored in acid washed and sample rinsed 15 ml polyethylene tubes. Samples were analyzed immediately or stored few hours at 4°C in the dark.

Analytical procedure : (briefly, could be a short recall to a published reference):

Nitrate, nitrite, orthophosphate and orthosilicic acid concentrations were determined on a Bran+Luebbe AutoAnalyzer 3 using standard colorimetric methods adapted for the analyzer (Grasshoff et al. 1999).

Units: µmol l-1

Sensor Precision: Analytical detection limit of 0.02 µmol l-1 for NO2, 0.03 µmol l-1 for NO3, 0.05 µmol l-1 for PO4 and 0.1 µmol l-1 for Si.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : Database ready.

### REFERENCES BIBLIOGRAPHIQUES

Grasshoff et al. (1999) Methods of seawater analyses, Weinheim, New-York