# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name (list of the measured parameters):

- ASD HDRF (Hemispherical-Directional Reflectance Factor)

- CAMLUM Lu (Upwelling radiance)

- CAMLUM HRDF (Hemispherical-directional Reflectance Distribution Function)

- OCR Ed (Downwelling Irradiance)

- AOD (Aerosol Optical Depth), RH (Relative Humidity), Angstrom Parameter

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   	GreenEdge – ICECAMP – 2015	LEG : 4

Date begin : 22/05/2015

Date end : 06/07/2015

Chief Scientist:  Simon Bélanger

Address :

Université du Québec à Rimouski

Département de biologie, chimie et géographie

300, allée des Ursulines

Rimouski (Québec), Canada

G5L 3A1

### OPERATION (if Relevant)

Sampling method :

Station number-Cast number :

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| Simon Bélanger | UQAR, 300 Allées des Ursulines, Rimouski, Québec, G5L 3A1 | +1 418 723-1986 ext. 1969 | (418) 724-1849 | simon_belanger@uqar.ca |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| Simon Bélanger and Clémence Goyens | UQAR, 300 Allées des Ursulines, Rimouski, Québec, G5L 3A1 | +1 418 723-1986 ext. 1969 (Simon Bélanger) |  | simon_belanger@uqar.ca clemence.goyens@takuvik.ulaval.ca |
| Edouard Leymarie | # LOV, UMR CNRS 7093 181 chemin du Lazaret 06230 Villefranche sur Mer (France) | # +33 (0)493763719 | # +33 (0)493763739 | # leymarie@obs-vlfr.fr |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

(1)

Predefined site (if relevant): Qikiqtarjuak – Ice Camp

Location: GreenEdge 2015 – IceCamp

LATITUDE: 67.479

LONGITUDE: -63.791

(2)

Predefined site (if relevant): Qikiqtarjuak – Ice floes near village

Location: GreenEdge 2015 – Qikiqtarjuak Village

LATITUDE: 64.034

LONGITUDE: -67.56

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

(1)

Instrument Type:

hyperspectral field spectrometer ASD

Manufacturer:

ASD Inc.

Model:

ASD FieldSpec Dual instrument #9106

Instrument Features / Calibration:

hyperspectral (350-1000 nm) radiance (10° field of view) and cosine irradiance sensors (raw data only –  not calibrated)

(2)

Instrument Type:

Fish-eye multispectral radiance camera CAMLUM

Manufacturer:

CIMEL Electronics in collaboration with Laboratoire d’Océanographie de Villefranche sur Mer (LOV)

Model:

CE600 SN

Instrument Features / Calibration:

Calibrated (LOV) radiance measurement at 406, 438, 494, 510, 560 and 628 nm over a full hemisphere (184° field of view) (Antoine et al., 2013)

(3)

Instrument Type:

SATLANTIC OCR IRRADIANCE SENSOR

Manufacturer:

SATLANTIC

Model:

Satlantic OCR SN 286

Instrument Features / Calibration:

Calibrated (LOV) cosine irradiance sensor at 406.42, 434.48, 442.33, 489.70, 508.43, 560.22 and 624.20 nm

(4)

Instrument Type:

Handheld 5-channel Sunphotometer Microtops II

Manufacturer:

Solar Light Company, Inc.

Model:

Microtops II SN 03766

Instrument Features / Calibration:

Calibrated spectral measurements at 440, 500, 675, 870 and 936 nm of the direct solar radiation for the estimation of columnar aerosol optical depth (AOD), water vapor and Angstrom parameter.

DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling:

- ASD Lu and Ed: The radiance sensor was placed on a goniometer to measure the upwelling radiance (Lu) over a homogeneous circular snow or ice surface (~ diameter of 8 cm at nadir) at zenith angles of 0°, 30° and 60° and azimuth angles from 0° (pointing towards the sun) to 330° at 30° intervals. The irradiance sensor was mounted on a ~1.5 m high tripod to measure the downwelling irradiance (Ed). Upwelling radiance measurements were also done over a white spectralon surface at start and end of each ASD measurement set. For each configuration, measurements were repeated 5 to 6 times.

- CAMLUM Lu: the CAMLUM was placed on a 4.20 m long pole with a handle at the end and a bridle in the middle to hold the camera. The pole was held by two persons along a 10 a 20 m long transect to measure the upwelling radiance (Lu) over a homogeneous snow or ice surface.

- OCR Ed : The irradiance sensor was placed on a ~ 1.5 m tripod to record the downwelling irradiance simultaneously with the CamLum Lu measurements.

Figure 1 presents a schematic overview of the field set-up to measure the anisotropy of the surface with both the ASD and the CamLum.

- Microptops : During clear days only, AOD measurements were done with the Microptops pointing towards the sun. A GPS was connected to it to get exact location and time. Each measurement was repeated 4 a 6 times.

Analytical procedure : (briefly, could be a short recall to a published reference):

- ASD HDRF: Hemispherical Directional Reflectance Factor (HDRF) was estimated for each surface with the ASD spectrometer as follows:

were LDN , EDN, ITL and ITE are the raw radiance and irradiance measurements and the integration time for the radiance and irradiance sensor, respectively, Θs is the solar zenith angle and Rspec is the  reflectance factor measured over the Lambertian white-grey spectralon surface for each station at the start and the end of a complete scan (measurements for a single surface over the entire hemisphere). Rspec is defined as:

where Cspec is the reflectance of the grey-white spectralon plaque (here 0.88).

The Hemispherical-directional Reflectance Distribution Function (HRDF) can be approximated by dividing HDRF by π. Note that in the literature the here defined reflectance quantity HRDF is often erroneously called “Bi-directional Distribution Function (BRDF)”. The terminology HDRF is chosen here as it is solely an approximation of the BRDF. Indeed, BRDF can not be measured on field as it does not correct the total reflectance signal for the contribution of the hemispherical diffuse irradiance (for more details see, among others, Schaepman-Strub et al., 2006).

- CAMLUM HRDF: The HRDF is measured using the calibrated radiance data from the multispectral CAMLUM and the irradiance data from the calibrated multispectral OCR.

Interpolation from the OCR to the CAMLUM spectral bands was performed according to Gregg and Carder (1990). The HDRF of a surface is defined as the median HDRF of all images taken along a 10 a 20 m long transect or circular area covered by spectrally homogeneous surfaces of snow or ice.

- Data from the microtops were sent to Sasha Smirnov at GSFC-NASA and processed following the procedure described in Smirnov et al (2009) to retrieve the aerosol optical thickness and angstrom exponent.

All data are accessible on line at : http://aeronet.gsfc.nasa.gov/new_web/maritime_aerosol_network.html

Units:

- ASD HDRF (none)

- CamLUM Lu (uW.cm^-2.sr^-1.nm^-1)

- CamLum HRDF (sr-1)

- OCR Ed (uW.cm^-2.nm^-1)

- AOD (none)

- RH (%)

- Angstrom Parameter (none)

Sensor Precision:

- ASD HDRF (none)

- CamLUM Lu (XX)

- OCR Ed (1-2%)

- Microtops (1-2%)

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery :

- ASD HDRF and CamLum HRDF : March 2016?

- AOD, RH and Angstrom Parameter: January 2016

### REFERENCES BIBLIOGRAPHIQUES

Antoine, D., Morel, A., Leymarie, E., Houyou, A., Gentili, B., Victori, S., Buis, J.-P., Buis, N., Meunier, S., Canini, M., Crozel, D., Fougnie, B., and Henry, P. (2013). Underwater radiance distributions measured with miniaturized multispectral radiance cameras, Journal of Atmospheric and Oceanic Technology, 30(1), 74-95.

Gregg, W. W., and Carder, K. L. (1990). A simple spectral solar irradiance model for cloudless maritime atmospheres, Limnology and Oceanography, 35(8), 1657-1675.

Perovich, D. (1994) Light reflection from sea ice during the onset of melt, Journal of

Geophysical Research, 99 (C2), 3351-3359.

Schaepman-Strub, G., Schaepman, M., Painter, T., Dangel, S., and Martonchik, J. (2006). Reflectance quantities in optical remote sensing – defintions and case studies, Remote Sensing of Environment, 103, 27-42

Smirnov, A., B. N. Holben, I. Slutsker, D. M. Giles, C. R. McClain, T. F. Eck, S. M. Sakerin, A. Macke, P. Croot, G. Zibordi, P. K. Quinn, J. Sciare, S. Kinne, M. Harvey, T. J. Smyth, S. Piketh, T. Zielinski, A. Proshutinsky, J. I. Goes, N. B. Nelson, P. Larouche, V. F. Radionov, P. Goloub, K. Krishna Moorthy, R. Matarrese, E. J. Robertson, and F. Jourdin (2009), Maritime Aerosol Network as a component of Aerosol Robotic Network,
Journal of Geophysical Research, 114, D06204, doi:10.1029/2008JD011257.