# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Concentration of dissolved amino acids (DAA)

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   	GreenEdge – ICECAMP – 2016	LEG :

Date begin : 2016-127 (Julian day)

Date end : 2016-204 (Julian day)

Chief Scientist: Marcel Babin

Address :

Université Laval

UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

### OPERATION (if Relevant)

Sampling method :Niskin bottle

Station number-Cast number :  First priority (165-204 in Julian day), Second priority (127-162 in Julian day)

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # A. Matsuoka | # Takuvik | # +1 418 656 2131 #3198 |  | # Atsushi.Matsuoka@takuvik.ulaval.ca |
| # R. Benner | # U. South Carolina | # +1 803-777-9561 |  | # BENNER@mailbox.sc.edu |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # J. Ferland | # Takuvik | # +1 418 656 2131 #8831 |  | # Joannie.Ferland@takuvik.ulaval.ca |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: Polar heaven ice camp site

LATITUDE: 67˚ 28.784N

LONGITUDE 63˚47.372W

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type: High performance liquid chromatography (HPLC) system equipped  with a fluorescence detector

Manufacturer: Agilent

Model:

Instrument Features / Calibration:

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Water column

Analytical procedure : (briefly, could be a short recall to a published reference):

Water samples were collected directly from a Niskin bottle into an acid-clean 60 ml HDPE Nalgene bottles through pre-combusted GF/F filters (450 ˚C for four hours). In total, 107 samples were collected for DAA analyses (portion of water samples collected for DOC will be used for the DAA determination). Those samples were then stored at -20 ˚C within 4-5 hours after the sampling. After the field campaign, all samples were shipped first to Takuvik and then to the University of South Carolina (USC) for analysis (arrived at USC on 22 February). Concentrations of DAA will be measured using an agilent high performance liquid chromatography (HPLC) system equipped with a fluorescence detector (Excitation: 330 nm; Emission:450 nm) installed at University of South Carolina according to Shen et al. (2012).

Units: nM

Sensor Precision:

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery :

### REFERENCES BIBLIOGRAPHIQUES

Shen, Y., C. G. Fichot, and R. Benner (2012), Dissolved organic matter composition and bioavailability reflect ecosystem productivity in the Western Arctic Ocean, Biogeosci., 9, 4993-5005, doi:10.5194/bg-9-4993-2012.