# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name (list of the measured parameters):  Sea ice Salinity

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   GreenEdge – ICECAMP – 2016

Date begin : 2 May 2016

Date end : 8 July 2016

Team Responsible: V. Galindo (PI: S. Rysgaard)

Address :

Centre for Earth Observation Science (CEOS), Faculty of Environment, Earth and Ressources,

University of Manitoba, Wallace building, local 495

Winnipeg R3T 2N2, Manitoba (CANADA)

OPERATION (if Relevant)

Sampling method :

Station number-Cast number :

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # GALINDO # Virginie # (Rysgaard) | # U. of Manitoba, Wallace building | # +1 204 474 9980 |  | # virginie.galindo@umanitoba.ca |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # GALINDO # Virginie # (Rysgaard) | # U. of Manitoba, Wallace building | # +1 204 474 9980 |  | # virginie.galindo@umanitoba.ca |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: Qiqiktarjuak ice camp

LATITUDE: 67°28.6’ N

LONGITUDE: 63°46.9’ W

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type: Orion portable salinometer

Manufacturer: Thermo Scientific

Model: WP-84TPS

Instrument Features / Calibration: Before measuring the bulk salinity on the sea ice samples, the salinity was measured on a standard solution of 35 followed by MilliQ water. These measurements were also done at the end. With these values, the data were corrected.

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling:

One full ice core was collected at the site during each ice core sampling day using a Kovacs Mark IV (9cm diameter) coring system. The collected ice core was cut in 10 cm sections, as quickly as possible to avoid brine drainage. The sections of sea ice were placed into Whirlpack bags to be transported back to the laboratory to be melted. Following the melting at room temperature, the salinity of each melted core was measured 24h after the sampling.

Analytical procedure : (briefly, could be a short recall to a published reference):

the bulk salin-

ity of these samples was measured using a calibrated Thermo

Scientific Orion portable salinometer WP-84TPS meter with

a precision of

0.1

Units: no unit

Sensor Precision: 0.1

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : Available

### REFERENCES BIBLIOGRAPHIQUES

Eicken, H. (2003), From the microscopic to the macroscopic to the regional scale, growth, microstrucutre and properties of sea ice, in Sea Ice - An introduction to its physics, biology, chemistry and geology, edited by B. Science, pp. 22–81.