# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Particulate  nutrients

Particulate organic carbon (POC)

Particulate organic nitrogen (PON)

Particulate organic phosphorus (POP)

PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   	GreenEdge – ICECAMP – 2015	LEG :

Date begin : 19/05/16

Date end :  08/07/16

Chief Scientist: Marcel Babin

Address :

Université Laval

UMI Takuvik

1045 avenue de la médecine

Québec, QC, G1V0A6, Canada

### OPERATION (if Relevant)

Station number-Cast number :

Operation code :

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Raimbault | # M.I.O. - Marseille | # 33 (4) 86090539 |  | # Patrick.raimbault@mio.osupytheas.fr |

### DATASET contact

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Raimbault | # M.I.O.- Marseille | # 33 (4) 86 090539 |  | # Patrick.raimbault@mio.osupytheas.fr |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: Ice camp, Polarhaven sampling, ice core sampling

LATITUDE:  67 28.784N

LONGITUDE: 063 47.372W

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type:

Manufacturer:

Model:

Instrument Features / Calibration:

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling and Analytical procedure :

Water samples or melted ice were collected in 1.2l polycarbonate bottles and immediately filtered onto 25 mm Whatman GF/F filters precombusted at 500°c during 4 hours. Filters were then placed in small glass tubes (also precombusted at 500°C) and dried at 60°. Filters were stored dried until analysis.

Analysis

Particulate organic carbon, nitrogen and phosphorus are measured at the Marseille laboratory using the wet-oxidation procedure described by Raimbault et al.(1999).

Units and Precision:

Particulate carbon in µmolesC.l-1: detection limit= 0.2 µmoles.l-1   accuracy = ± 0.1µmoles.l-1

Particulate nitrogen in µmolesN.l-1: detection limit= 0.05 µmoles.l-1   accuracy = ± 0.05µmoles.l-1

Particulate phosphorus in µmolesP.l-1: detection limit= 0.02 µmoles.l-1   accuracy = ± 0.01µmoles.l-1

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : august 2017

### REFERENCES BIBLIOGRAPHIQUES

Raimbault P., Diaz F., Boudjellal B., 1999 Simultaneous determination of particulate forms of carbon, nitrogen and phosphorus collected on filters using a semi-automatic wet-oxidation procedure. Mar. Ecol. Progr. Ser., 180: 289-295.