META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | November 16, 2009 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | November 16, 2009 | last update |

| ### PARAMETRES CONCERNES | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage | No parametre | nom parametre | Status : [non] valid |
| --- | --- | --- | --- |
| Sampling | parameter number | name of the parameter | Status : [not] valid |
| AOP Package on Amundsen (BioSORS) | 1 2 | Water-Leaving Radiance Global Solar Irradiance | Valid Valid |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

Characterization of the Apparent Optical Properties (IOP) of the Arctic in the Beaufort Sea and Mackenzie River

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Stan Hooker | NASA CVO Maryland | # 410-533-6451 |  | # Stanford.b.hooker@nasa.gov |
| # James Brown | # NASA CVO Maryland | # 305-421-4959 |  | # Jim.Brown@miami.edu |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

The main objective of the sampling strategy is to characterize the apparent optical properties of the sea surface.

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

AOPs with Biospherical, Inc. instrumentation :

1) BioSORS 19 channel Lt, Li, and Es sensors (320-780nm).

Data was collected using a proprietary data aquisition system and software to collect the data in real time using a combined sea and sky plus a solar reference cable connected to a Macintosh computer.

# Stratégie d'échantillonnage /

# Sampling strategy

AOPs were measured from the Amundsen with a system mounted to a platform on the bow plus a surface reference.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

-Data Processing and corrections applied

-Time frame is dependent on the availability of the pigment data (approximately 6-9 months)

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

Precision of the instrumentation is less than 0.1% and the accuracy in the field is less than 5%.

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

Not available yet.

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

Not available yet.

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

Not available yet.

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |

Not available yet.