META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | 16/07/2010 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | 16/07/2010 | last update |

| ### PARAMETRES CONCERNES | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage | No parametre | nom parametre | Status : [non] valid |
| --- | --- | --- | --- |
| Sampling | parameter number | name of the parameter | Status : [not] valid |
| Filtration and lab. analysis |  |  |  |
| /barge (BRG) | (2347) (2140) | Particulate Organic Carbon (POC) SPM (Suspended Particulate Material) | Not valid |
| /Rosette | (30044) (30053) | Particulate Organic Carbon (POC) SPM (Suspended Particulate Material) | Not valid |
| / zodiac ( | (2348) (2141) | Particulate Organic Carbon (POC) SPM (Suspended Particulate Material) | Not valid |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

MALINA

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Babin Marcel | LOV-CNRS | # 04 93 76 37 12 | # 37 39 | # marcel@obs-vlfr.fr |
| # Doxaran David | LOV-CNRS | # 04 93 76 37 24 | # 37 39 | # doxaran@obs-vlfr.fr |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

How do changes in ice cover, permafrost and UV radiation impact on biodiversity and biogeochemical fluxes in the Arctic Ocean?

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |
| Suspended Particulate Matter concentration | SPM (mg/L) |
| Particulate Organic Carbon concentration | POC (mg/L) |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

SPM: filtration through pre-combusted and pre-weighed Whatman GF/F filters of a certain volume of the water sample collected, then dry weight was determined less than 2

months later in the laboratory (van der Linde 1998).

POC: (i) acidification of the SPM filter with HCl-2N (volume adapted to the composition and mass of particles retained on the filter based on King et al. (1998)) to remove carbonated then

(ii) Combustion of the filter on a pre-calibrated CHN analyzer at LOV (Perkin Elmer 2400 Series II CHNS/O Analyzer) to determine the mass of organic carbon contained by the particles.

# Stratégie d'échantillonnage /

# Sampling strategy

Water samples collected using:

Niskin bottles from the rosette onboard the N/O amundsen

Bottles just below the water surface, onboard the barge and the Zodiac

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

3 months

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

Standard deviations provided + intercomparison exercice was carried out to estimate the uncertainties on POC

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

SPM&POC_MALINA_Legs_1b_2b_4.xls

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

STATION = station name	        CAST = cast number

DEPTH (m) = depth where was collected the water sample

BOTTLE = bottle number on the rosette (BRG if barge)

Latitude = in decimal unit	         Longitude = in decimal unit

SPM (mg/L) = selective mean value from triplicates

SPM sd (mg/L) = corresponding standard deviation

POC (mg/L) = selective mean value from triplicates

POC sd (mg/L) = corresponding standard deviation

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

Relationships between SPM and POC and rganic carbon content of SPM (POC/SPM ratio in %)

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |

KING et al. 1998. Analysis of total and organic carbon and total nitrogen in settling oceanic particles and a marine sediment: an interlaboratory comparison. Marine Chemistry, 60. 203–216.

VAN DER LINDE, D. W. 1998. Protocol for determination of total suspended matter in oceans and coastal zones. JRC Technical Note I.98.182.