# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name :  	Chromophoric dissolved organic Matter (CDOM) m-1

### PROJET-ETUDE / PROJECT TITLE & Chefs de projet et mission / PIs

Campaign  NAME : OUTPACE  		LEG :

Date begin : 18/02/2015

Date end : 03/04/2015

Chief Scientist: Thierry Moutin & Sophie Bonnet

Address : M.I.O. Institut Méditerranéen d’Océanologie - UMR 7294 
OSU Institut Pythéas, Campus de Luminy, Bâtiment Méditerranée 
13288 MARSEILLE cedex 09, FRANCE

& M.I.O. Institut Méditerranéen d’Océanologie - IRD/CNRS/Aix-Marseille University

IRD Noumea, 101 Promenade R. Laroque, BP A5, 98 848 Nouméa Cedex

NEW CALEDONIA

Chief Mission : Thierry Moutin

Address : M.I.O. Institut Méditerranéen d’Océanologie - UMR 7294 
OSU Institut Pythéas , Campus de Luminy, Bâtiment Méditerranée 
13288 MARSEILLE cedex 09, France

### ECHANTILLONNAGE ET OPERATIONS A LA MER / SAMPLING METHOD AND OPERATIONS AT SEA

Sampling method : Rosette

Station number-Cast number :

| Station Number | Sampled Cast Number |
| --- | --- |
| SD-01 | OUT-006 ; OUT-007 |
| SD-02 | OUT-010 ; OUT-016 |
| SD-03 | OUT-019 ; OUT-020 |
| LD-A | OUT-027 ; OUT-033 ; OUT-041; OUT-050; OUT-061 ; OUT-063; OUT-067 |
| SD-04 | OUT-070 ; OUT-071 |
| SD-05 | OUT-074 ; OUT-075 |
| SD-06 | OUT-078 ; OUT-079 |
| SD-07 | OUT-082 ; OUT-083 |
| SD-08 | OUT-086 ; OUT-087 |
| SD-09 | OUT-091 ; TMC-012 |
| SD-10 | OUT-094 ; OUT095 |
| SD-11 | OUT-098 ; OUT-099 |
| SD-12 | OUT-102 ; OUT-103 |
| LD-B | OUT-109 ; OUT-127 . OUT-145 ; OUT-150 OUT-151 |
| SD-13 | OUT-152 |
| LD-C | OUT-158 ; OUT-163 ; OUT-175 ; TMC 18 . OUT-193 ; OUT-194 . OUT-198 ; OUT-199 |
| SD-14 | OUT-209 , OUT-210 |
| SD-15 | OUT-212 ; OUT-213 |

Operation code :  aCDOM (m-1)

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / # phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Dupouy Cécile | Aix-Marseille Université, Univ. Toulon, CNRS, IRD UMR235 (UM110) Centre IRD Nouméa BP A5 - 98848 Nouméa cedex Nouvelle Calédonie | 687 26 07 29 |  | # Cecile.dupouy@mio.osupytheas.fr |

Remarque / Remark : Pour la pérennité de la base de données, il est fortement suggéré que le responsable scientifique ait un poste permanent / For the perinity of the database, it is recommended that the PI of the parameter has a permanent position.

### Contact Base de données pour ce paramètre / DATASET contact for this parameter

| Remarque / Remark : Pour la pérennité de la base de données, il est fortement suggéré que le contact base de donnée ait un poste permanent / For the perinity of the database, it is recommended that the dataset contact for this parameter has a permanent position. # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # DUPOUY Cécile | Aix-Marseille Université, Univ. Toulon, CNRS, IRD UMR235 (UM110) Centre IRD Nouméa BP A5 - 98848 Nouméa cedex Nouvelle Calédonie | 687 26 07 29 |  | # Cecile.dupouy@mio.osupytheas.fr |

### Autre(s) participant(s) à la mesure de ce paramètre / Other participant(s) for the measurement of this parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # role | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Röttgers, Rüdiger | Institute of Coastal Research Helmholtz-Zentrum Geesthacht Centre for Materials and Coastal Research Max-Planck-Str. 1 D-21502 Geesthacht tel +49 (0)4152 87 1575 fax +49 (0)4152 87 1596 | +49 (0)4152 87 1575 |  | # rroettgers@hzg.de |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):

Location: South Pacific Ocean

LATITUDE:  S 17° - S 23°

LONGITUDE: E 159° – W 149°

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION

Instrument Type: Ultrapath LWCC (Liquid Waveguide Capillary Cell) + fibres optiques WPI + spectromètre Hamamatsu

Manufacturer: WPI

Model: LWCC-2200

Instrument Features / Calibration:

Calibration is performed with nigrosin prepared by diluting a stock solution of Nigrosin in 0.2 µm filtered MilliQ water.

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: : Samples were collected from the Niskin bottles in combusted glass bottles with Teflon liner caps rinsed with HCL5% and then MQ water, and were immediately filtered using a Nalgene filtration unit through a cellulose ester MILLIPORE 0,22μm 47mm membrane filter (GSWP04700) rinsed before with 200 ml MQ. For filters rich in chlorophyll, a first filtration was performed through pre-combusted GF/F filters (Whatman) under low vacuum (<100 mbar) to avoid breakage of phytoplankton cells during the filtration. Filtered samples were always kept at the same temperature as the reference (ultra pure water) to minimize temperature effects. Filtered samples were then measured on board within 1-3 hours after sampling in the 2m LWCC in triplicates.

Analytical procedure: (briefly, could be a short recall to a published reference):

Spectrophotometric analysis was performed using the UltraPath 200 cm liquid waveguide cell (World Precision Instruments, Sarasota, FL, USA; Miller et al., 2002), a DH-2000 UV-Vis lamp, an UV/Vis (200-800nm High sensitivity type) Hamamatsu compact TM-MiniSpectrometer, (C10082CA)  and a personal computer with HZG-LWCC Python routine (from HZG, R. Röttgers). Absorbance spectra were measured with reference to 0.2 µm filtered Milli-Q water. A salinity correction was then applied, obtained from a measurement of a NaCl solution. Between measurements, cell was flushed with Milli-Q water until the base line is recovered. An air bubble was introduced to separate each sample. The CDOM absorption coefficient aCDOM(L) in m-1 was determined by using the relationship:

aCDOM = 2.303A(L)/Length

where A(L) is the absorbance at wavelength (L) and Length the pathlength (here 2m). Spectra were fitted considering a typical exponential increase of the CDOM spectrum with decreasing wavelength with the slope S and the absorption calculated as in Bricaud et al., 2010. Linear fits to log-transformed aCDOM data were used to calculate S in different spectral ranges as in Sempéré et al. (2015).

Units:   m-1

Sensor Precision:

Typical analytical precision is 1.6% at 280 nm 1.9% at 300 nm, 3.1 % at 400nm and 0.2 % at 500 nm.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : First Trimester 2016

### REFERENCES BIBLIOGRAPHIQUES

- Bricaud, A., Babin, M., Claustre, H., Ras, J. and F. Tieche, 2010. Light absorption properties and absorption budget of South East Pacific waters. Journal of Geophysical Research, VOL. 115, C08009, doi:10.1029/2009JC005517.

- Murakami, H., Dupouy C., 2013. Atmospheric correction and inherent optical property estimation in the southwest New Caledonia lagoon using AVNIR-2 high-resolution data, Applied Optics Vol. 52, Iss. 2, pp. 182–198

- Röttgers, R., and Doerffer, 2007, R., “Measurements of optical absorption by chromophoric dissolved organic matter using a point-source integrating-cavity absorption meter,” Limnology and Oceanography Methods 5, 126-135.

- Röttgers, R, and Koch, B.P., 2012. “Spectroscopic detection of a ubiquitous dissolved pigment degradation product in subsurface waters of the global ocean”, Biogeosciences, 9, 2585–2596.

- Röttgers, R., Dupouy, C., Taylor, B., Bracher, A., and S. Wozniak, 2014. Mass-specific light absorption coefficients of natural aquatic particles in the near-infrared spectral region, Limnol. Oceanogr., 59(5), 2014, 1449–1460.

- Röttgers, R., Doxaran, D., Dupouy, C., 2016. Quantitative filter technique measurements of spectral light absorption by aquatic particles using a portable integrating cavity absorption meter (QFT-ICAM).  Optics Express 24, 2, DOI:10.1364/OE.24.0000A1

- Sempere, R., Para, J., Tedetti, M., Charrière, B., and M. Mallet, 2015. Variability of solar radiation and CDOM in surface coastal waters of the Western Mediterranean Sea. Photochemistry and Photobiology, 91, 851-861.