INSTRUMENT/METHOD .....Optical sensor.. 	IDENTIFIER : (given by the data center)

NAME : Underwater Video Profiler (U.V.P.2a)

Description of the main instruments/methods used at sea and in the laboratory during the project.

Examples : CTD, Current meter, salinometer, sediment trap, spectrometer ...

Laboratory (Owner of the instrument or who performs the method ):

Laboratoire d’Océanographie Biologique et Ecologie du Plancton Marin (LOBEPM)

Address :

Station Zoologique

BP 28

06234 Villefranche sur mer

Tel : 0493763816	Fax : 0493763834

Email : Gorsky@ccrv.obs-vlfr.fr

Contact Scientist :

### Gabriel GORSKY

INSTRUMENT NAME (60) : Underwater Video Profiler 2a (U.V.P.2a) commonly called PVM2

MANUFACTURER (60) : LOBEPM

MANUFACTURING DATE (10) : 1989

SERIAL NUMBER (20) : 2a

DESCRIPTION  (240) :

Version 2a of the UVP. Built for the study of MARINE SNOW and ZOOPLANKTON.

TECHNICAL CHARACTERISTICS (240) :

1000 m operational depth

In situ recordings at 25 Hz

1 CCD V700E Sony camera (adjustable lens)

Marine snow mode with structured lights.

92 µm ESD lower limit detection.

> 202 µm ESD lower limit of size measurements.

Zooplankton mode with 150 W, 300 W or 400 W spots

COMMENTS (120)

INSTRUMENT TYPE (circle the main type) :

In situ Sensor (default)	Transmitter/Receiver	On board recorder

Drifter	Towed platform	Expendable sensor

OTHER ATTACHED EQUIPEMENT (in case of complex multi sensor:Paltform equipment) (10) :SBE19 – SBE911 (according to the cruise)

DATE  (of updating this form) : 07-30-26

| #### MEASURED PARAMETER 1 | CALIBRATION DATE | CRUISES | COMMENTS |
| --- | --- | --- | --- |
| Zoom mode for particles > 92 ESD µm |  | Dynaproc | LOBEPM |

BIBLIOGRAPHICAL REFERENCES (METHODOLOGY) :

Gorsky, G., Aldorf, C., Picheral, M., Kage, M., Garcia,Y. and J. Favole (1992) Vertical distribution of suspended aggregates determined by a new Underwater Video Profiler. Ann. Inst. oceanogr., Paris, 68 (1-2): 275-280.

Gorsky, G., Picheral, M. and L. Stemmann (in press) Use of the Underwater Video Profiler for the Study of Aggregate Dynamics in the North Mediterranean. Estuarine, Coastal and Shelf Science.

Picheral, M., Grisoni, J-M., Stemmann, L. and G. Gorsky (1998) Underwater Video Profiler for the "in situ" study of suspended particulate matter. OCEANS 98, 28 September- 1 October, IEEE/OES conference, Nice,  p. 171-174.

Picheral, M., Stemmann. L. et G. Gorsky (1995) Système multiparamètrique pour la mesure et la quantification de la matière particulaire en suspension dans la colonne d'eau. 3e Colloque Européen Des Capteurs pour l'Environnement, Grenoble, 30-31 Mars 1995. pp. 162-165.

Stemmann, L. (1998) Analyse spatio-temporelle de la matière particulaire. Thèse Doctorale, Université Paris 6, pp 178.

Stemmann, L., Picheral, M. and G. Gorsky. (in press) Diel changes in the vertical distribution of suspended particulate matter in the NW Mediterranean Sea investigated with the Underwater Video Profiler. Deep-Sea Research.