CRUISE REPORT – Leg 1b

Edouard Leymarie, LOV Villefranche-sur-mer, France

MEASUREMENTS OF LIGHT ABSORPTION

BY COLORED DISSOLVED ORGANIC MATTER (CDOM) AND PARTICULATE MATTER

- Objectives during Leg 1B: preparation of the MALINA cruise

- Test of the system in the present environment

- define a robust protocol for the measurement, including water temperature problem (huge difference between sea water and reference’s temperature)

- First view of the spatial variations (in amplitude and spectral dependence) of CDOM absorption coefficients over the transect.

- Comparison of results with those obtained with the ULTRAPATH system for the CDOM measurement and with the integrating sphere for the particulate absorption.

- Sampling

15 (over 28) samples have been collected underway, from the pumping system or with a bucket. Sampling was performed three times per day (9:00, 13:00, 18:00) from July 5 (18:00) to July 14 (18:00).

Samples were collected on the same carboy (A) as those collected for pigments, particulate absorption, SPM/POC, and ULTRAPATH measurements.

- Measurements

Light absorption spectra of CDOM and Particulate matter have been measured for each sample between 350 and 750 nm by using a PSICAM prototype developed in Villefranche. This technique allows the measurement of very low absorption coefficient with a very weak scattering dependency. For each sample two measurements are done. The first after a filtration at 0.2 µm gives the CDOM absorption coefficient. The second, without filtration, gives the total absorption coefficient. The difference of these coefficients gives the absorption of particles.