META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | 09/11/2009 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | 16/12/2010 | last update |

| ### PARAMETRES CONCERNES | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage | No parametre | nom parametre | Status : [non] valid |
| --- | --- | --- | --- |
| Sampling | parameter number | name of the parameter | Status : [not] valid |
| CTD-optical frame deployed from the barge |  |  |  |
|  | p2441 p2444 AC9-serial 279 | Light attenuation and absorption at 9 visible wavelengths | Valid |
|  | p2442 p2445 AC9- serial 303 | Light attenuation and absorption at 9 visible and near-infrared wavelengths | Valid |
|  | p2447 ECO-BB3 serial 028 | Light backscattering at 3 visible wavelengths | Valid |
|  | P2446 ECO-BB3 serial 538 (720nm -770nm- 870nm) | Light backscattering at 3 near-infrared wavelengths | Valid (not at 870 nm) |
|  | p2516 p2505 p2457 | CTD : T(z), S(z), Cond(z) | Valid |
|  | p2451 | Chl-a fluorometer Chelsea mini-tracka II | Valid |
|  | p2546 | CDOM fluorometer Wetlabs wetstar WSCD | Valid |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

MALINA : bio-optical characterization of  coastal waters o the Beaufort Sea influenced by the Mackenzie River plume

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # David Doxaran | LOV – CNRS / UPMC | # 04 93 76 37 24 | # 04 93 76 37 39 | # doxaran@obs-vlfr.fr |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

The objective is to characterize the bio-optical properties of the Beaufort Sea coastal waters influenced by the Mackenzie River plume as a function of the concentrations of suspended (algla and non-algal particles, particulate organic carbon) and dissolved (colored dissolved organic matter, CDOM) materials. These field measurements are used to develop regional algorithms for ocean color remote sensing. Field and satellite ocean color products will then be used to estimate the fluxes of terrigeneous substances exported by the Mackenzie River into the Beaufort Sea.

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

One CTD sensor, one CDOM fluorometer, one chlorophyll-a fluorometer, two Wetlabs AC-9 meters and two Wetlabs ECO-BB3 backscattering meters have been used utilisés to measure along the water column (0 – 35 m depth): the water temperature, pressure and salinity; the light attenuation, absorption and backscattering coefficients of the water in the visible and near-infrared spectral domains.. The optical data have been corrected for temperature, salinity, absorption (ECO-BB3) and residual scattering (AC-9) effects.

# Stratégie d'échantillonnage /

# Sampling strategy

All the sensors were mounted on a frame which was deployed from the barge along the water colum (typically from 0 to 35 m depth). After a 5 min. warm-up, the casts down and up were recoreded then the data were averaged every meter.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

The CTD eand bio-optical data recorded on the onboard the R/V Amundsen will be useful for intercomparison with the barge data. The light absorption coefficients measured in the ab. (Ultrapath and Lambda-19 data) will be used for intercomparison with AC-9 measurements.

The final CTD-IOP data from the barge should be delivered in January 2011 into the MALINA database.

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

Data have been processed based on updated protocols (Wetlabs User’sguide 2007, 2009). Ongoing study taking into account the calibration and sensitivity of each sensor, together with correctionsapplied to the measurements.

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

One text file per per run cast recorded by the datalogger. The correspondence between the run casts and the Malina sampling stations is provided hereafter.

res3_archive_”runcast”_1m.txt

Station

691, 691, 692, 680, 394, 394, 280, 280, 260, 260, 220, 220, 240, 240, 240, 170, 170, 150, 150, 380, 380, 360, 360, 320, 320, 340, 340, 670, 670, 660, 660, 620, 620, 760, 760, 780, 695, 694, 697, 696, 691, 345_1, 345_1, 345_2, 398, 397, 396, 395, 540, 540, 430, 430, 460, 135_1, 135_2, 135_2, 235, 235_2, 235_2

Run cast

136, 137, 138, 139, 140, 141, 155, 158, 159, 160, 161, 162, 163, 164, 166, 168, 169, 171, 172, 174, 175, 176, 177, 178, 179, 180, 181, 182, 183, 185, 186, 187, 188, 189, 190, 191, 192, 193, 194, 195, 196, 197, 198, 199, 200, 201, 202, 203, 204, 205, 206, 207, 208, 209, 210, 211, 212, 213, 214

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

En tête actuelle des colonnes de données:

"Time(ms)","Pres(dbar)","Temp(C)","Cond(S/m)","Sal(PSU)","a_412","a_440","a_488"

,"a_510","a_532","a_555","a_630","a_676","a_715","c_412","c_440","c_488","c_510","c_532","c_555","c_630","c_676","c_715","slope_cvis","R2_cvis","a_440","a_555","a_630","a_715","a_730","a_750","a_767","a_820","a_870","c_440","c_555","c_630","c_715","c_730","c_750","c_767","c_820","c_870","slope_cnir","R2_cnir","bbp_440","bbp_532","bbp_650","bbp_720", "bbp_770","bbp_870"

With:

Pres(dbar),Temp(C),Cond(S/m),Sal(PSU) measured with the Seabird FastCat

fluo_Chl	(V) the chlorophyll-a fluorescence measured with the Chelsea MiniTracka II sensor (Serial Number: 175-217)

fluo_cdom (V) the CDOM fluorescence measured with the Wetlabs WetStar WSCD-877P

a = absorption, in m-1, the first nine columns are values measured with the Wetlabs AC9-279, the second nine columns are values measured with the Wetlabs AC9-0303

c = attenuation in m-1, the first nine columns are values measured with the Wetlabs AC9-279, the second nine columns are values measured with the Wetlabs AC9-0303

bbp = backscattering in m-1, measured with the Wetlabs ECO-BB3-028 (440, 532 and 650 nm) and the Wetlabs ECO-BB3-538 (720, 770 and 870 nm).

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

The preliminar results have been presented at the Ocean optics XX conference at Anchorage, Alaska (USA) in October 2010 (Doxaran et al. 2010).

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |

Doxaran D., Babin M., Chami M., Lorthiois T., Ruddick K. and D. McKee (2010). Visible and near-infrared spectral variations of light backscattering by particles in coastal waters. Ocean Optics XX conference, Anchorage (USA, AK), 27 Sept. – 1 Oct. 2010.

Wetlabs Absorption and Attenuation Meter (ac-9) User’s Guide (2009). Wetlabs Inc., May 2009.

Wetlabs Scattering Meter ECO-BB User’s Guide (2007). Wetlabs Inc., September 2007.