FICHE  META_INFORMATION_PARAMETRES

DYNAPROC 2

### PARAMETRES CONCERNES  (1 ligne par paramètre)

NO3

NO2

NH4

PO4 Si(OH)4

Chla fluo

PC Particulate carbon

PN Particulate nitrogen

PP Particulate phosphorus

TOC total organic carbon

TON total organic nitrogen

TOP total organic phosphorus

DOC dissolved organic carbon

DON dissolved organic nitrogen

DOP dissolved organic phosphorus

Primary production 13C

Nitrate uptake 15N

Ammonium uptake 15N

N2 fixation 15N

### OPERATION & CAMPAIN

PECHE

DYNAPROC 2

### PROJET  ETUDE / PROJECT TITLE

Impact of the water column structure on new production and export flux.

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # RAIMBAULT Patrick | # CNRS UMR 6535 Campus de Luminy 13288 Marseille | # 33 4 91 82 91 05 | # 33 4 91 82 19 91 | # raimbault@com.univ-mrs.fr |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

The main objectives are :

- Quantify the time evolution of nutrients and biomass

- Quantify primary production, nitrogen assimilation and export of particulate matter in relation to the vertical structure o f the water column

- Importance of regeneration processes in carbon and nitrgoen recycling

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Macronutrients

Collection and storage of samples

Water samples were collected at different depth according to the water mass stratification and location of chlorophyll maximum. Water was transferred from the Niskin bottles with a Teflon tube into 100 ml litres polyethylene bottles previously washed with chloride acid 2N and rinsed with MilliQ water before the cruise. 500 µl of mercuric chloride were added to poison the samples which stored at dark until analysis at laboratory.

Analysis

Analysis were performed on board and at the laboratory using an automated colorimetric procedure (Tréguer and LeCorre, 1975) for nitrate, nitrite, phosphate and silicate and fluoresence method for ammonium (Holmes et al., 1999).

Inorganic carbon and nitrogen assimilation

Collection and incubation of samples

Six water samples were collected at different depth between surface and 60 m depth. Water was transferred from the Niskin bottles with a Teflon tube into three 600 ml polycarbonate bottles. After addition of 13C tracer (H13CO3 mg.l-1), samples were spiked with inorganic nitrogen labelled with 15N. One sample was spiked with 100µl of 15N-NH4 (1µmole.ml-1), a second with 50µl of 15N-NO3 (1µmole.ml-1); the third sample was spiked with 1 ml of 15N2 gaz. After tracer addition, samples were placed into different plastic boxes covered by a nickel screen simulating different light intensity corresponding to the depths of sampling. Temperature in incubators was maintained by continuous circulation of surface water. Incubation was stopped at sunset and samples were immediately filtered on Whatman GF/F filters (25 mm in diameter) precombusted at 450°C. Filters were dried at 60°C during12 hours and stored dry until mass-spectrometer analysis. Filtrates were poisoned with 1 ml HgCl2 (6g.l-1) and stored until analysis

Analysis

Filtrates were treated as proposed by Slawyk and Raimbault (1995) and Raimbault et al. (1999) to quantify ammonium regeneration, nitrification and dissolved organic nitrogen. 13C/ 15N enrichments were measured on a TracerMass  mass-spectrometer.

| - Instrument (si besoin) | - Paramètres | Méthode | Observations |
| --- | --- | --- | --- |

# Stratégie d'échantillonnage / Sampling strategy

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

Nitrate in µmoles.l-1: detection limit= 0.05 µmoles.l-1   accuracy = ± 0.05µmoles.l-1

Nitrite in µmoles.l-1: detection limit= 0.03 µmoles.l-1   accuracy = ± 0.03µmoles.l-1

Phosphate in µmoles.l-1: detection limit= 0.02 µmoles.l-1   accuracy = ± 0.05µmoles.l-1

Silicates in µmoles.l-1: detection limit= 0.05 µmoles.l-1   accuracy = ± 0.05µmoles.l-

Nitrogen uptake in nmolesN.l-1 .d-1:   accuracy = ± 0.5 nmolesN.l-1 .d-1

Primary production in µmolesC.l-1 .d-1:   accuracy = ± 0.5 µmolesC.l-1 .d-1

### FICHIERS / FILES

# Nom de fichier de données / file name

Afin de lever le doute ou toute confusion dans le cas de paramètres identiques, spécifier aussi l’appareil ou la méthode.

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

| Macronutrients Noms et abréviations utilisées |  |
| --- | --- |
| Nitrates NO3 Nitrite NO2 Phosphates PO4 Silicates SiOH4 |  |
| Inorganic carbon and nitrogen assimilation Noms et abréviations utilisées |  |
| Nitrate uptake NO3 Ammonium uptaken NH4 Nitrogen fixation N2 Primary Production PP |  |

### RESULTATS PRELIMINAIRES

### REFERENCES BIBLIOGRAPHIQUES

Tréguer, P., LeCorre, P., 1975. Manuel d’analyses des sels nutritifs dans l’eau de mer (Utilisation de l’Autoanalyser II), 2ème edn. Laboratoire. de Chimie Marine, Université de Bretagne Occidentale, Brest, 110 pp.

Holmes R.M .,Aminot A., Kerouel R., Hooker B.A., Peterson B.J., 1999. A simple and precise method for measuring ammonium in marine and freshwater ecosystems. Can J. Fish. Aquat. Sci., 56: 1801-1808

Slawyk G., Raimbault P.,1995. A simple procedure for the simultaneous recovery of dissolved inorganic and organic nitrogen in 15N-tracer experiments on oceanic waters improving the mass balance. Mar. Ecol. Prog. Ser.,124: 289-299

Raimbault P., Slawyk G., Boudjellal B., Coatanoan C., Conan P., Coste B., Garcia N., Moutin T., Pujo-Pay M., 1999. Biomass, new production and export in the equatorial Pacific at 150°W: Evidence for intense nitrogen recycling. J. Geophys. Res. 104, 3341-3356.

Raimbault P., Slawyk G., Garcia N, 2000. Comparison between chemical and isotopic measurements of biological nitrate utilization: further evidence of low new production levels in the equatorial Pacific. Mar. biol., 136 :1147-115