# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre)

### Nom du DATASET / Data SET NAME

Data set Name : Particulate backscattering coefficient (bbp)

### PROJET-ETUDE / PROJECT TITLE

Campaign  NAME :   SOCLIM	LEG :

Date begin : 6/10/2016

Date end : 1/11/2016

Chief Scientist: Stéphane Blain

Address :	Laboratoire océanographie microbienne

Observatoire océanologique de banyuls sur mer

66650 banyuls sur mer

Chief Mission :  Stéphane Blain

Address :	idem

### OPERATION (if Relevant)

Sampling method : WETLabs ECO Triplet “BBFL2” instrument attached onto a CTD-rosette system.

Station number-Cast number : O22 – O23 – O24 – O25 – TNS06 – TB1 – TB2 – A3_1 – A3_2 – KERFIX – FS – O11 – SI – TNS08 – O12

Operation code : CTD002 – CTD003 – CTD004 – CTD005 – CTD006 – CTD007 – CTD008 – CTD009 – CTD010 – CTD012 – CTD013 –CTD014 – CTD015

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Julia Uitz | # Laboratoire d’Océanographie de Villefranche (LOV) # 181 Chemin du Lazaret # 06230 Villefranche sur Mer | # 04 93 76 38 73 | # 04 93 76 37 39 | # julia.uitz@obs-vlfr.fr |
| # Hervé Claustre | # Laboratoire d’Océanographie de Villefranche (LOV) # 181 Chemin du Lazaret # 06230 Villefranche sur Mer | # 04 93 76 37 29 | # 04 93 76 37 39 | # claustre@obs-vlfr.fr |

### DATASET contact

| # Nom / name | # adresse / address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Julia Uitz | # Laboratoire d’Océanographie de Villefranche (LOV) # 181 Chemin du Lazaret # 06230 Villefranche sur Mer | # 04 93 76 38 73 | # 04 93 76 37 39 | # julia.uitz@obs-vlfr.fr |

### INFORMATION GEOGRAPHIQUES /  GEOGRAPHIC INFORMATION

Predefined site (if relevant):  Indian Ocean and Indian sector of the Southern ocean

Location:

LATITUDE:  15°S – 58.5 °S

LONGITUDE 55°E-75°E

### DESCRIPTION DES INSTRUMENTS /  INSTRUMENTS DESCRIPTION (if Relevant)

Instrument Type: ECO Triplet optical sensor

Manufacturer: WET Labs Inc.

Model:  The ECO Triplet “BBFL2” measures the chlorophyll a fluorescence (excitation/emission wavelengths: 470 nm/695 nm), the colored dissolved organic matter (CDOM) fluorescence (excitation/emission wavelengths: 370 nm/460 nm) and the backscattering coefficient of particles at 700 nm.

Instrument Features / Calibration:

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: The ECO Triplet sensor collects high-frequency (xxxx) optical measurements within the water column from the surface to xxxx m….

Analytical procedure:

The backscattering sensor of the ECO Triplet provides the volume scattering function (VSF) at an angle of 124° relative to the direction of light propagation, at a wavelength of 700 nm, β(124, 700) in units of sr-1 m-1. This quantity is obtained through the conversion of the raw counts from the sensor using the dark counts and calibration coefficient provided by the manufacturer.

The VSF β(124, 700) includes the contribution of particles and seawater molecules to scattering. The VSF of particles, βp(124, 700), is retrieved by subtracting the VSF of seawater, βsw(124, 700) from β(124, 700). The βsw(124, 700), that depends on temperature and salinity, is obtained following Zhang et al. (2009).

The particulate backscattering coefficient at an angle of 124° and a wavelength of 700 nm, bbp(700) in units of m-1, is determined using a conversion factor  following Boss and Pegau (2001) and Sullivan and Twardowski (2009):

bbp(700) = 2  βp(124, 700)

with a value of of 1.076 (Sullivan and Twardowski 2009).

The vertical profiles of bbp(700) are then quality controlled by checking the range of values following the technical specifications from the manufacturer (WET Labs, 2016) and removing negative spikes following Briggs et al. (2011).

Units: The backscattering coefficient of particles, bbp, derived from the raw signal from the backscattering sensor has units of m-1

Sensor Precision:

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this :

Estimated Date of Delivery: March 2017

### REFERENCES BIBLIOGRAPHIQUES

Boss E., & Pegau W. S. (2001). Relationship of light scattering at an angle in the backward direction to the backscattering coefficient. Applied Optics, 40(30), 5503–5507.

Briggs N., Perry M.J., Cetinić I., Lee C., D’Asaro E., Gray A.M., & Rehm E. (2011). High-resolution observations of aggregate flux during a sub-polar North Atlantic spring bloom. Deep-Sea Research I, 58, 1031–1039.

Sullivan, J.M., & Twardowski M.S. (2009). Angular shape of the oceanic particulate volume scattering function in the backward direction. Applied optics, 48(35), 6811–9, doi:10.1364/AO.48.006811.

WET Labs (2016). ECO Fluorometers and Scattering Sensors. User manual eco160114, Edition B, 34 pp.

Zhang X., Hu L., & He M.X. (2009). Scattering by pure seawater: Effect of salinity. Optics Express, 17(7), 5698, doi :10.1364/OE.17.005698.