FICHE  META_INFORMATION_PARAMETRES

(à remplir par le responsable du paramètre)

mise à jour fiche / Last update : 18-nov-2005

### PARAMETRES CONCERNES

N°99125

Ba-dis-filtered-ros

### OPERATION & CAMPAGNE / OPERATION & CAMPAIN

Nom de l’opération: BARMED

Nom des campagnes : BARMED 1, 2, 3, 4 and 5

### PROJET  ETUDE / PROJECT TITLE

BARMED: BARium in the MEDiterranean Sea.

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # JEANDEL catherine | # LEGOS (UMR 5566) | # 335 61332933 | # 33561253205 | # Catherine.Jeandel@cnes.fr |
|  | # OMP 14 Ave E Belin 31400 Toulouse |  |  |  |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

#### In the framework of BARMED, the goals related to the study of the parameter “dissolved barium in seawater” (noted above: “Ba-dis-ros”) are:

  - to establish the seasonal variability of the dissolved barium vertical profiles on aliquots of FILTERED seawater collected at about 24 depths of the water column per cruise.

  - To compare this variation to other relevant parameters established in the framework of BARMED (eg particulate barium, barite and Ba/Sr determined by SEM) or in the framework of SODYF (e.g: macronutrients and primary production)

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Aliquots of 2 ml of filtered seawater have been spiked using a spike enriched at 97% in 135 Ba. Given the natural abundance of 138Ba and 135Ba (138Ba/135Ba=10.8767) and previous estimations of Ba concentrations in the Mediterranean Sea, 135Ba-enriched spike was added to samples in quantities so as to have a final 138Ba/135Ba ratio of ~1 in a total sample volume of 10 ml.

Samples were then diluted up to a volume of 10 ml adding MQ water enriched in HNO3 (final HN03 concentration in the sample: 2% in mass), and measured by the isotopic dilution method using the ICP/MS Perkin Elan 6000 equipped with a micronebulizer. The blank signal was indistinguishable from the background signal of the ICP/MS.

Stratégie d'échantillonnage / Sampling strategy

Aliquots of 30 ml of seawater samples were collected just after the filtration on Nuclepore membranes (47mm diameter; 0,45 m porosity) of ~10 L of seawater to harvest the suspended particles, at the exit of the PERPEX system (see the filtration protocol for parameter # 118):

| ##### Sampling depths (meters) |  |  |  |  |
| --- | --- | --- | --- | --- |
| BARMED 1 | BARMED 2 | BARMED 3 | BARMED 4 | BARMED 5 |
| 5 10 20 30 (max Chl a) 50 70 110 150 200 250 300 | 0 10 15 (max Chl a) 25 75 100 185 (small part maximum) 250 300 360 (large part maximum) 400 500 600 1000 1500 2000 2200 2300 | 0 10 20 30 (max Chl a) 50 75 100 150 200 250 300 350 | 0 112 300 500 600 1000 2200 | 0 15 48 (max Chl a) 100 150 200 250 300 350 500 600 700 900 1000 2300 |

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

The following data will be useful to achieve the interpretation of these data : hydrological parameters (including nutrients), pigments, primary production, total suspended barium, suspended barite and Ba/Sr and Ba/Ca on the barites of the same samples; bioSi; suspended and dissolved Ra and Th isotopes, phytoplankton and zooplankton taxonomies at selected depths.

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

Rosette dissolved filtered Ba concentrations

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

For each cruise (Barmed 1 to Barmed 5), dissolved Ba (filtered) concentrations are given in nmol (10-9 mol) per kg of seawater versus depth (in meters). Errors are given in the same unit.

Barmed 1: 15/02/2003

Barmed 2: 23-24/03/2003

Barmed 3: 09-10/04/2003

Barmed 4: 15-16/05/2003

Barmed 5: 24-25/06/2003

### RESULTATS PRELIMINAIRES  / RESULTS

### REFERENCES BIBLIOGRAPHIQUES

(séparer d’une ligne blanche chaque article)