FICHE  META_INFORMATION_PARAMETRES

(à remplir par le responsable du paramètre)

### PARAMETRES CONCERNES

N°99106

Ba-dis-unfiltered-ros

### OPERATION & CAMPAGNE / OPERATION & CAMPAIN

Nom de l’opération: BARMED

Nom des campagnes : BARMED 1, 2, 3, 4 and 5

### PROJET  ETUDE / PROJECT TITLE

BARMED: BARium in the MEDiterranean Sea.

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # JEANDEL catherine | # LEGOS (UMR 5566) | # 335 61332933 | # 33561253205 | # Catherine.Jeandel@cnes.fr |
|  | # OMP 14 Ave E Belin 31400 Toulouse |  |  |  |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

#### In the framework of BARMED, the goals related to the study of the parameter “dissolved barium in seawater” (noted above: “Ba-dis-unfiltered-ros”) are:

  - to establish the seasonal variability of the dissolved barium vertical profiles on aliquots of UNFILTERED seawater collected at about 24 depths of the water column per cruise. Since the particulate Ba pool is insignificant compared with the dissolved Ba pool, total Ba (what is measured in unfiltered seawater) is commonly referred to as dissolved Ba.

  - To compare this variation to other relevant parameters established in the framework of BARMED (eg particulate barium, barite and Ba/Sr determined by SEM) or in the framework of SODYF (e.g: macronutrients and primary production)

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Aliquots of 2 ml of unfiltered seawater were spiked using a spike enriched at 97% in 135 Ba. Given the natural abundance of 138Ba and 135Ba (138Ba/135Ba=10.8767) and previous estimations of Ba concentrations in the Mediterranean Sea, 135Ba-enriched spike was added to samples in quantities so as to have a final 138Ba/135Ba ratio of ~1 in a total sample volume of 10 ml.

Samples were then diluted up to a volume of 10 ml adding MQ water enriched in HNO3 (final HN03 concentration in the sample: 2% in mass), and measured by the isotopic dilution method using the ICP/MS Perkin Elan 6000 equipped with a micronebulizer. The blank signal was indistinguishable from the background signal of the ICP/MS.

Stratégie d'échantillonnage / Sampling strategy

Aliquots of 30 ml  of seawater samples were collected using Niskin bottles at the following depths :

| ##### Sampling depths (meters) |  |  |  |  |
| --- | --- | --- | --- | --- |
| BARMED 1 | BARMED 2 | BARMED 3 | BARMED 4 | BARMED 5 |
| 5 10 20 30 (max Chl a) 50 70 110 150 200 300 350 400 500 600 1000 1500 2000 | 0 10 15 (max Chl a) 25 75 100 185 (small part maximum) 250 300 360 (large part maximum) 400 500 600 1000 1500 2000 2200 2300 | 0 10 20 30 (max Chl a) 50 75 100 150 200 300 350 | 0 10 30 50 (max Chl a) 60 75 112 140 (small part maximum) 210 240 (large part maximum) 300 350 1000 1500 2000 2200 2300 | 0 15 30 75 100 150 200 250 300 350 400 500 600 700 800 900 1000 1500 2000 2300 |

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

The following data will be useful to achieve the interpretation of these data : hydrological parameters (including nutrients), pigments, primary production, total suspended barium, suspended barite and Ba/Sr and Ba/Ca on the barites of the same samples; bioSi; suspended and dissolved Ra and Th isotopes, phytoplankton and zooplankton taxonomies at selected depths.

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

Rosette dissolved unfiltered Ba concentrations

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

For each cruise (Barmed 1 to Barmed 5), dissolved Ba (unfiltered) concentrations are given in nmol (10-9 mol) per kg of seawater versus depth (in meters). Errors are given in the same unit.

Barmed 1: 15/02/2003

Barmed 2: 23-24/03/2003

Barmed 3: 09-10/04/2003

Barmed 4: 15-16/05/2003

Barmed 5: 24-25/06/2003

### RESULTATS PRELIMINAIRES  / RESULTS

### REFERENCES BIBLIOGRAPHIQUES

(séparer d’une ligne blanche chaque article)