FICHE  META_INFORMATION_PARAMETRES

(à remplir par le responsable du paramètre)

### PARAMETRES CONCERNES

N°

99113	Al-susp-ros - AAS

99111	Sr-susp-ros

118	Ba-susp-ros

99108	Mn-susp-ros

99109	Th-susp-ros

99112	Ca-susp-ros - AAS

119	Barites and elements - SEM

### OPERATION & CAMPAGNE / OPERATION & CAMPAIN

Nom de l’opération: BARMED

Nom des campagnes : BARMED 1, 2, 3, 4 and 5

### PROJET  ETUDE / PROJECT TITLE

BARMED: BARium in the MEDiterranean Sea.

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # JEANDEL catherine | # LEGOS (UMR 5566) | # 335 61332933 | # 33561253205 | # Catherine.Jeandel@cnes.fr |
|  | # OMP 14 Ave E Belin 31400 Toulouse |  |  |  |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

The specific goal of these measurements is to compare the seasonal variability of suspended biogenic Ba concentrations (and other associated elements such as Sr, Mn, REE, Th….) analyzed after a destructive acid digestion and using an ICP-MS (Elan 6000) analysis to the seasonal variability of other relevant parameters established in the framework of BARMED (e.g Ra isotopes, Th isotopes but also the taxonomy for ex.)  or in the framework of SODYF (e.g: macronutrients, primary and export production). In addition, the Ba and Sr associated to barite and celestite will be assessed by SEM measurements and compared to the total Ba (Sr) profiles obtained here.

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Two disks representing about 9% of the nuclepore filter each, have been prealably cut to allow the independent measurements of i) the barite and Sr/Ba in barite by SEM and ii) biogenic silica on the same filters.

Concentrations of the elements listed above have been determined after a total acid of the remaining fraction of the nuclepore filter was performed as follows: filters were covered with 6 ml of 6 M Q-HCl and 1 ml of concentrated Q-HNO3, heated on a hot plate for 90 min (110°C), and cooled in the freezer for 45 min. 0.2 ml of concentrated HF was then added, the same heating/cooling procedure following, and the resulting solution was let to evaporate. The digestion residue was diluted in 10 ml of HNO3 2%, an internal standard of In/Re (10 ppb) was added and the total concentrations of the elements (except Al and Ca) analysed using and ELAN 6000 Perkin Elemer ICP/MS. Samples were analysed in Al and Ca through Atomic Absorption Spectrometry (AAS) after having been diluted to suit the concentration range of the spectrometer. Specifically, flame AAS was used for Ca, and electrothermal AAS for Al.

Stratégie d'échantillonnage / Sampling strategy

About 10 l of seawater samples were collected using Niskin bottles at the following depths :

| ##### Sampling depths |  |  |  |  |
| --- | --- | --- | --- | --- |
| BARMED 1 | BARMED 2 | BARMED 3 | BARMED 4 | BARMED 5 |
| 5 10 20 30 (max Chl a) 50 70 110 150 200 300 350 400 500 600 1000 1500 2000 | 0 10 15 (max Chl a) 25 75 100 185 (small part maximum) 250 300 360 (large part maximum) 400 500 600 1000 1500 2000 2200 2300 | 0 10 20 30 (max Chl a) 75 100 150 200 300 350 | 0 10 30 50 (max Chl a) 60 75 112 140 (small part maximum) 210 240 (large part maximum) 300 350 400 500 600 1000 1500 2000 2200 2300 | 0 15 30 48 (max Chl a) 75 100 150 200 250 300 350 400 500 600 700 800 900 1000 1500 2000 2300 |

Within 24 to 48 h after sampling, these volumes have been filtered using 8 PERSPEX filtration systems borrowed from F. Dehairs. Those are polycarbonates cylinders at the base of which nuclepore filters (47mm diameter, 0.45 m porosity) are mounted in a filter holder: on the top, introduction of filtered air ensures an overpressure; at the bottom (after the filter holder)  filtered water is pumped using a  peristaltic system.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

The following data will be useful to achieve the interpretation of these data : hydrological parameters (including nutrients), pigments, primary production, suspended barite and Ba/Sr on the barites of the same samples; bioSi; suspended and dissolved Ra and Th isotopes, phytoplankton and zooplankton taxonomies at selected depths

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

Rosette suspended particle concentrations

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

For each cruise (Barmed 1 to Barmed 5), total particulate concentrations - i.e. not corrected from the lithogenic fraction - of Ba, Sr, Mn, Th, Al and Ca are given, in nmol (10-9 mol) or pmol (10-12 mol) per liter of seawater versus depth (in meters). Errors are given in the same units.

Barmed 1: 15/02/2003

Barmed 2: 23-24/03/2003

Barmed 3: 09-10/04/2003

Barmed 4: 15-16/05/2003

Barmed 5: 24-25/06/2003

### RESULTATS PRELIMINAIRES  / RESULTS

### REFERENCES BIBLIOGRAPHIQUES

(séparer d’une ligne blanche chaque article)