FICHE  META_INFORMATION_PARAMETRES

( A renseigner AVANT la mission / Fill before the cruise )

### PARAMETRES CONCERNES / PROJECT TITLE
(1 ligne par paramètre / 1 line per parameter)
(Indiquer en premier le numéro du paramètre / Firt indicate the number of the parameter)

83: Zooplankton  (Meso-), Stereo Microscope

### OPERATION & CAMPAIN

BIOSOPE

### PROJET  ETUDE / PROJECT TITLE

Percentage of density distributions of major mesozooplankton taxa sampled by plankton nets (> 200µm) at 9 stations along the BIOSOPE transect

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom /name | # adresse /address | # Téléphone/Phone | # fax /fax number | # mél /email |
| --- | --- | --- | --- | --- |
| # François Carlotti | Rue de la Batterie des Lions F- 13007 Marseille FRANCE | # (33) 4 91 04 16 44 | # (33) 4 91 04 16 35 | # carlotti@com.univ-mrs.fr |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF THE PROJECT

The main objectives are :

- Describe the zooplankton biodiversity and distribution pattern in relation with the physical and biological variables

(A compléter APRES la mission / Fill before the end of the cruise or after the cruise )

### DESCRIPTION DES PARAMETRES /  PARAMETER DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Zooplankton was sampled using a WP2 net fitted with 200 µm mesh-size towed vertically from 200 m to the surface at 1ms-1.

Samples were preserved directly upon collection (5% buffered formalin solution).

9 samples, one for each of the six long stations and one for each of the three transect sections, were quantitatively split with a Motoda box for taxonomic determination. Up to 300 to 500 specimens were identified to different taxa levels, until more than 250 individuals of the dominant group (copepods) were counted.

Spécifier selon les cas l’instrument. Préciser les méthodes ou références bibliographiques des méthodes

Tableau OU texte explicatif

| - Instrument (si besoin) | - Paramètres | Méthode | Observations |
| --- | --- | --- | --- |
| Taxonomic determination | Percentage of densities | See text |  |

# Stratégie d'échantillonnage / Sampling strategy

Zooplankton was sampled using a WP2 net fitted with 200 µm mesh-size towed vertically from 200 m to the surface at 1ms-1.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

Afin de lever le doute ou toute confusion dans le cas de paramètres identiques, spécifier aussi l’appareil ou la méthode.

Taxonomic percentage in 9 stations.xls

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

### RESULTATS PRELIMINAIRES

### REFERENCES

(séparer par d’une ligne blanche chaque article)

- Leborgne, R., Champalbert G., Gaudy R. (2003). Mesozooplankton biomass and composition in the equatorial Pacific along 180°. Journal of Geophysical Research, 108 (C12), 8143, DOI 10.1029/2000JC000745.

Riandey, V., Champalbert, G., Carlotti, F., Taupier-Letage, I., Thibault-Botha, D., 2005. Zooplankton distribution related to the hydrodynamic features in the Algerian Basin (western Mediterranean Sea) in summer 1997. Deep Sea Research I 52, 2029-2048.