FICHE  META_INFORMATION_PARAMETRES

( A renseigner AVANT la mission / Fill before the cruise )

### PARAMETRES CONCERNES / PROJECT TITLE
(1 ligne par paramètre / 1 line per parameter)
(Indiquer en premier le numéro du paramètre / Firt indicate the number of the parameter)

192	pN2O 	(SIMON instrument)

### OPERATION & CAMPAIN

BIOSOPE

### PROJET  ETUDE / PROJECT TITLE

Air-sea N2O flux  along  different oceanographic regimes in eastern South Pacific

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom /name | # adresse /address | # Téléphone/Phone | # fax /fax number | # mél /email |
| --- | --- | --- | --- | --- |
| # Laura Farías | Cabina 7 s/n. Barrio Universitario, Concepcion. Chile | # (56) 41-203738 | # (56) 41-239900 | # lfarias@profc.udec.cl |
| # Other responsible Osvaldo Ulloa | Cabina 7 s/n. Barrio Universitario, Concepcion. Chile | # (56) 41-203585 | # (56) 41-239900 | # oulloa@profc.udec.cl |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF THE PROJECT

To measure gradient of concentration  between the atmosphere and the surface water column) using continuous method along the proposed cruise track (BIOSOPE Cruise) in order to obtain a picture of air-sea N2O flux under different hydrography and oceanographic regions from the eastern South Pacific.

To correlate N2O air-sea flux with hydrographic and oceanographic variables and process in order to elucidate mechanisms of N2O production in the area.

(A compléter APRES la mission / Fill before the end of the cruise or after the cruise )

### DESCRIPTION DES PARAMETRES /  PARAMETER DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Continuous measurement of N2O in the surface sea water and in the atmosphere (air-sea gradient) using Simon instrument.

| - Instrument | - Paramètres | Méthode | Observations |
| --- | --- | --- | --- |
| SIMOM | pN2O | SIMON | Flux |

The measurement of nitrous oxide in the atmosphere, sea water and standard is done by electron capture chromatography using a chromatograph Shimadzu GC-17. The gas sample is injected into the chromatograph through the Automatic Injection System (AIS) which can be physically separated in three components: the “Control Unit”, the “Valve module” and the “Instrumental Panels”.  All components are part of SIMON "Measurement System of Nitrous Oxide, that was developed at PROFC-Universidad de Concepción

# Stratégie d'échantillonnage / Sampling strategy

The underway N2O measurement

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Post analysis will be required after to collect the data in conjunction with CTD and some meteorological data

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

Afin de lever le doute ou toute confusion dans le cas de paramètres identiques, spécifier aussi l’appareil ou la méthode.

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

### RESULTATS PRELIMINAIRES

SIMOM was used throughout the BEAGLE cruise (JAMSTEC) .

### REFERENCES

Villagran, 2004: http://www.profc.udec.cl/~victor/trabajo/trabaj_sim.htm