FICHE  META_INFORMATION_PARAMETRES

( A renseigner AVANT la mission / Fill before the cruise )

### PARAMETRES CONCERNES / PROJECT TITLE
(1 ligne par paramètre / 1 line per parameter)
(Indiquer en premier le numéro du paramètre / Firt indicate the number of the parameter)

414: UVR underwater profiles and surface irradiance

218: PAR underwater profiles and surface irradiance

### OPERATION & CAMPAIN

BIOSOPE

### PROJET  ETUDE / PROJECT TITLE

Penetration of UVR/PAR in the water column of the South East Pacific.

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom /name | # adresse /address | # Téléphone/Phone | # fax /fax number | # mél /email |
| --- | --- | --- | --- | --- |
| # Richard Sempéré | # LMGEM UMR 6117 Campus de Luminy 13288 Marseille | # (33) 4 91 82 93 61 | # (33) 4 91 82 90 51 | # sempere@com.univ-mrs.fr |
| # Mac Tedetti | # LMGEM UMR 6117 Campus de Luminy # 13288 Marseille | # (33) 4 91 82 93 50 | # (33) 4 91 82 90 51 | # tedetti@com.univ-mrs.fr |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF THE PROJECT

To study the importance and the variability of UVR (A and B) and PAR intensity at the ocean surface and in the water column (0-200 m) in the South East Pacific.

To compare in situ UV measurements with UV penetration derived from models (that use TOMS and SeaWiFS data products) in the framework of collaboration between R. Sempéré and A. Vasilkov (Science Systems and Applications, Lanham, Maryland, USA).

(A compléter APRES la mission / Fill before the end of the cruise or after the cruise)

### DESCRIPTION DES PARAMETRES /  PARAMETER DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

| - Instrument (si besoin) | - Paramètres | Méthode |
| --- | --- | --- |
| MicroPro Profiler | UV/PAR Downwelling irradiance (Ed) and upwelling radiance (Lu) | Free Fall deployment |
| Surface reference | UV/PAR surface irradiance (Es) |  |

# Stratégie d'échantillonnage / Sampling strategy

During the cruise, we want to do at least 3 UV/PAR profiles (0-100 m, around solar noon, with measurement of surface irradiance simultaneously) per day depending on the meteorological conditions. In addition, we will use the surface irradiance during our deck incubations. The surface sensors will be mounted to the vessel and free fall deployments will be done from the stern if possible.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

### RESULTATS PRELIMINAIRES

### REFERENCES