META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | 08/10/2008 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | 09/10/2008 | last update |

#### ( AVANT la mission )	( BEFORE the cruise)

| ### PARAMETRES CONCERNES Chlorophyll-a: chl-a Phaeopigments: phaeo | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage |  | No parametre |  | nom parametre |  | Status : [non] valid |
| --- | --- | --- | --- | --- | --- | --- |
| Sampling |  | parameter number |  | name of the parameter |  | Status : [not] valid |
| NISKIN bottles | (39) Chla Fluorescence (Turner) Stock Boye M./Masson A./Guenneugues A. |  | Chlorophyll-a |  | valid |  |
| NISKIN bottles | (1041) Phaeo Fluorescence (Turner) Stock Boye M./Masson A./Guenneugues A |  | Phaeopigment |  | valid |  |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

BONUS-GOODHOPE

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Marie Boye | LEMAR CNRS UMR6539, IUEM, 29280 Plouzane | 33 2 98 49 86 51 | 33 2 98 49 86 45 | marie.boye@univ-brest.fr |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

Determination of the chlorophyll-a and phaeopigments (ie, degradation product of chl-a) distributions at depth along the transect of Bonus Goodhope from Capetown (SA) towards 57°S along the Greenwich Meridian.

(APRES  la mission ) (AFTER the cruise)

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

……………

| Method description: |  |
| --- | --- |
| Chl-a analysis | (Strickland and Parsons, 1972; Evans et al, 1987) |
|  | Filtration on GF/F filters |
|  | Fluorimetric measurements with a TURNER Design 10-AN |
|  | Calibration made before and during the cruise from spinach solution (Sigma) |
| Limits of detection: | 0,005 µg/L |

| Method description: |  |
| --- | --- |
| Phaeopigments analysis | (Strickland and Parsons, 1972; Evans et al, 1987) |
|  | Filtration on GF/F filters |
|  | Fluorimetric measurements with a TURNER Design 10-AN |
|  | concentration obtained from Chl-a concentration following Lorensen (1966) |
| Limits of detection: | 0,005 µg/L |

# Stratégie d'échantillonnage /

# Sampling strategy

……………

The top water-column has been sampled and analysed for the HYDRO stations, as well as the “MLD-casts” at LARGE and SUPER stations.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

……………

To properly interpret the data-sets, the water-masses need to be identified (ie, T, S, density are requested). To discuss the data-sets additional biogeochemical parameters would be used (such as nutrients, pigments, taxonomy, nutritive trace metals, pCO2…).

The samples were analysed on board and the data are validated.

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

……………

see in 5.1.

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

| (39) Chla Fluorescence (Turner) Stock Boye M./Masson A./Guenneugues A. | Chlorophyll-a |
| --- | --- |
| (1041) Phaeo Fluorescence (Turner) Stock Boye M./Masson A./Guenneugues A | Phaeopigment |

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

| Title | Description | units |
| --- | --- | --- |
| Date of analysis | On board (13/02/08 to 24/03/08) | dd/mm/yyyy |
| Cruise or Project | BONUS-GOODHOPE |  |
| Latitude (°) | Latitude | degrees decimal |
| Longitude (°) | Longitude | degrees decimal |
| Date | date of sampling | dd/mm/yyyy |
| CTD | CTD number |  |
| Bottle | Niskin bottle number |  |
| Depth (m) | Depth | metres |
| Chl-a | Chlorophyll-a concentration | µg/L |
| Phaeo | Phaeopigments concentration | µg/L |

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

See the cruise report.

An article is in preparation since sept. 2008.

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |