META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | 07/10/2008 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | 07/10/2008 15/12/2009 (mptorre) 17/12/2010 (mptorre) (b) | last update |

#### ( AVANT la mission )	( BEFORE the cruise)

| ### PARAMETRES CONCERNES Nitrate: NO3 Silicate: Si(OH)4 | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage |  | No parametre |  | nom parametre |  | Status : [non] valid |
| --- | --- | --- | --- | --- | --- | --- |
| Sampling |  | parameter number |  | name of the parameter |  | Status : [not] valid |
| NISKIN bottles | (28) NO3 Technicon Autoanalyser Stock Boye M./LeMoigne F./Masson A. |  | nitrate |  | Valid : last update : 2010-dec-17(b) |  |
| NISKIN bottles | (33) Si(OH)4 Technicon Autoanalyser Stock Boye M./LeMoigne F./Masson A. |  | silicate |  | valid |  |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

BONUS-GOODHOPE

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Marie Boye | LEMAR CNRS UMR6539, IUEM, 29280 Plouzane | 33 2 98 49 86 51 | 33 2 98 49 86 45 | marie.boye@univ-brest.fr |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

Determination of the nitrate and silicate distributions at depth along the transect of Bonus Goodhope from Capetown (SA) towards 57°S along the Greenwich Meridian.

(APRES  la mission ) (AFTER the cruise)

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

……………

| Method description: |  |
| --- | --- |
| Nitrate Analysis | The analytical procedure is described in: Treguer and Le Corre (1975) |
|  | Method: Technicon AutoAnalyseur III Bran&Luebbe |
|  | The base line was obtained using artificial water sea : NaHCO3 + MgSO4+ NaCl |
|  | Ranges of standards used for calibration: 0; 5; 10; 20; 30; 40 µM |
|  | Standard solution : KNO3 MERCK |
| Limits of detection (Technicon): | 0,05 µM |

| Method description: |  |
| --- | --- |
| SilicateAnalysis | The analytical procedure is described in: Treguer and Le Corre (1975) |
|  | Method: Technicon AutoAnalyseur III Bran&Luebbe |
|  | The base line was obtained using artificial water sea : NaHCO3 + MgSO4+ NaCl |
|  | 2 standards ranges were used: [0; 5; 10; 20; 50; 75 µM] and [75; 100; 125; 150 µM], depending of the depth and the stations position |
|  | The standard solution was dry sodium hexafluorosilicate from MERCK |
| Limits of detection (technicon): | 0,1 µM |

# Stratégie d'échantillonnage /

# Sampling strategy

……………

Almost all the CDT-Niskin casts were sampled and analysed for NO3 and Si(OH)4. This especially includes all the bottom CTD-Niskin rosette casts of the “hydrological” stations, “large” and “super” stations.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

……………

To properly interpret the data-sets, the water-masses need to be identified (ie, T, S, density are requested). To discuss the data-sets additional biogeochemical parameters would be used (such as chl-a, pigments, taxonomy, nutritive trace metals…).

The samples were analysed on board and the data are validated.

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

……………

see in 5.1.

# MISES à jour du jeu de données  /

# Data set UPDATES  [ mptorre update ]

……………

Wrong values in the CTD 32 regarding  nitrate parameter has been notified.

NEW  values are available into the file :  BGH_CTD_32_nitrate_corrected__2009-12-14

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

(28) NO3 Technicon Autoanalyser Stock Boye M./LeMoigne F./Masson A.

(33) Si(OH)4 Technicon Autoanalyser Stock Boye M./LeMoigne F./Masson A.

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

| Title | Description | units |
| --- | --- | --- |
| Date of analysis | on board (13/02/08 to 24/03/08) | dd/mm/yyyy |
| Cruise or Project | BONUS-GOODHOPE |  |
| Latitude (°) | Latitude | degrees decimal |
| Longitude (°) | Longitude | degrees decimal |
| Date | date of sampling | dd/mm/yyyy |
| Time | time of sampling | dd/mm/yyyy |
| CTD | CTD number |  |
| Bottle | Niskin bottle number |  |
| Depth (m) | Depth | metres |
| NO3 | Nitrate concentration | µM |
| Si(OH)4 | Silicate concentration | µM |

NOTE : the new file of December 17, 2010 lists only the NO3 parameter (b)

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

See the cruise report.

An article is in preparation since sept. 2008.

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |