META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | 31/03/08 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | 20/05/09 | last update |

| ### PARAMETRES CONCERNES | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage | No parametre | nom parametre | Status : [non] valid |
| --- | --- | --- | --- |
| Sampling | parameter number | name of the parameter | Status : [not] valid |
| above-water sensor, TRIOS radiometer | 495 | irradiance (downwelling) PAR | not valid |
| under-water sensor, TRIOS radiometer | 496 | radiance (upwelling) | not valid |
| Rosette – Cycle, WetLabs ECO-VSF | 601 | backscattering coefficient (1 l) | not valid |
| Rosette – Cycle, Spectrophotometry | 602 | Particulate absorption | not valid |
| Rosette – Cycle, Spectrophotometry | 615 | Dissolved absorption (CDOM) |  |
| Rosette – Cycle, Fluorescence | 683 | FLUO-CDOM | not valid |
| under-water sensor, TRIOS radiometer | 684 | Irradiance (downwelling) PAR | not valid |
| Rosette – Cycle, Cstar transmissometer | 685 | Attenuation coefficient 660 nm | not valid |
| Rosette – Cycle, Fluorescence | 686 | Chlorophyll-a concentration | not valid |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

BOUM

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Hubert Loisel | Laboratoire d'Océanologie et de Géosciences, Maison de la Recherche en Environnement Naturel, 32, avenue Foch 62930 Wimereux | # +33 (0)3 21 99 64 20 | # + 33 (0)3 21 99 64 01 | # hubert.loisel@univ-littoral.fr |
| Karin Nordkvist | Laboratoire d'Océanologie et de Géosciences MREN 32, avenue Foch 62930 Wimereux | # +33 (0)3 21 99 64 14 | # + 33 (0)3 21 99 64 01 | # karin.nordkvist@univ-littoral.fr |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

To study the diurnal variations of the optical properties of the seawater at the three long duration stations.

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

……………

495, Irradiance (downwelling) PAR: Downwelling irradiance above the sea surface, [W/(m² nm)]. Measured by a TriOS irradiance sensor mounted above the water and facing upwards. The sensor  has 190 channels in the range 320-950 nm. This sensor is on the deck, or better mounted on the vigi, far from any shadow and perturbation.

496, Radiance (upwelling): Upwelling radiance at different depths below the sea surface, [W/(m²

sr nm)]. Measured by a TriOS radiance sensor under water, facing downwards. The sensor has 190

channels in the range 320-950 nm.

601, backscattering coefficient:  The ECO-VSF measures the scattering at three angles: 100, 125,

and 150 degrees, thus providing the shape of the Volume Scattering Function (VSF) throughout the

backscattering region. This is done at two wavelengths: 660 and 532 nm. The signal at 532 nm was

too weak to give any reasonable values. The backscattering at 660 nm, bb_660, sometimes has

negative values and should be calibrated.

602, particulate absorption: Sampling for absorption measurements was done at the three long duration stations (A, B, C). Water samples were taken from the CTD rosette, bottles 09 (near the deep chlorophyll maximum, DCM) and 24, at around 5 m depth. Depending on the particle load in the water, 2.8 to 5.6 liters of water were filtered for each sample. Filtering was done soon after sampling, never more than 6 hours later. Bottles were stored dark and cool until filtering. The filters were put in liquid nitrogen directly after filtering, and then stored at -80° until analysis. Two blank filters were prepared each day by soaking them in 2µm filtered sea water, and then stored in the same way as the sample filters.

A Cary 100 UV-Vis double-beem spectrophotometer was used for the absorption measurements. In the beginning of each measurement session, a baseline was recorded using two blank filters. The spectrophotometer automatically performs the baseline correction for the sample filter spectra. After measurement of the total absorption by particles, the pigments were extracted in methanol and the de-pigmented absorption was measured. The absorption by pigments was calculated as the difference between these two.

Chlorophyll-specific absorption coefficients were obtained by normalizing for chlorophyll-a concentration, as measured by the rosette fluorometer and calibrated with HPLC profiles.

For details on sampling, analysis and calculations, see Mitchell et al (2003).

615, dissolved absorption: Not measured.

683, CDOM: Concentration of colored dissolved organic matter measured by an ECO Fluorometer

from WetLabs.

684, Irradiance (downwelling) PAR: downwelling radiance at different depths below the sea

surface, [W/(m² nm)]. Measured by a TriOS irradiance sensor under water, facing upwards. The

sensor has 190 channels in the wavelength range 320-950 nm.

685, attenuation coefficient at 660 nm: The beam attenuation coefficient, C (m–1), a measure of

the effective attenuation of light in the water column due to absorption and scattering, is measured

by the Cstar transmissometer from WetLabs. For each profile, the beam attenuation is calibrated by

subtracting the value at 400 m depth from the entire profile. This value is considered as being

representative of the value for particle-free water. The remaining part of the signal is attributed to

chlorophyll contaning particles (Loisel and Morel, 1998).

686, chlorophyll concentration: The concentration of chlorophyll-a measured by the

Chelsea Aquatracka fluorometer mounted on the rosette. One HPLC profile is available just after

the last cycle cast on each long duration station. This profile is used to find a calibration coefficient

by linear regression, so that the total chl-a concentration can be estimated as [chla] = a*fluo+b.

See Table 1 for details about the regression, and Figure 1 for graphs.

| Station | HPLC profiles used for the regression (CTD nr) | Linear regression | r2 |
| --- | --- | --- | --- |
| A | 127, 128, 130, 186, 187, 188, 190, 192, 193 | [tchla]=2.5263*fluo+0.0038 | 0.9683 |
| B | 002, 114, 115 | [tchla]=2.3575*fluo+0.0059 | 0.9771 |
| C | 016, 017, 071 | [tchla]=3.0638*fluo-0.0097 | 0.9812 |

Table 1. Linear regression between [tchla] from HPLC measurements and fluorescence data.

Figure 1. HPLC measured [tchla] plotted against fluorescence, together with the regression line.

# Stratégie d'échantillonnage /

# Sampling strategy

……………

Parameters 601, 602, 683, 685, 686 (backscattering coefficient, particulate absorption, CDOM

concentration, attenuation coefficient and chlorophyll-a concentration) are measured every three

hours on the long duration stations (CTD cast 0-500m).

615, dissolved absorption, is measured up to four times on each long duration station and used for

calibration of the CDOM fluorometer.

495, 496 and 684 (above-water downwelling irradiance, under-water upwelling radiance and

downwelling irradiance) are measured at the long duration stations.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

……………

602, particulate absorption: post-cruise spectrophotometry of the samples, mid september.

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

……………

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

BOUM_XXX_grappe_optique.xls, XXX is the CTD number. Parameters 601, 683, 685, 686.

BOUM_radiometer_XX.xls, XX is the number of the radiometric cast. Parameters 495, 496, 684.

ap.xls. Parameter 602, particulate absorption. Other parameters from the absorption measurements are saved in ad.xls, aphi.xls, ap_chl_spec.xls, aphi_chl_spec.xls, beta_fp.xls, beta_fd.xls, OD_fp.xls, OD_fd.xls.

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

For details, see the readme-tab in each file.

- BOUM_XXX_grappe_optique.xls

Line 1: id=BOUM_ctdXXX

XXX is the ctd number

Line 2: Parameters

| Column title | Description | Unit |
| --- | --- | --- |
| Time(ms) | Time from start of acquisition | ms |
| Pres(dbar) | Pressure | dbar |
| Temp(C) | Temperature | °C |
| Cond(S/m) | Conductivity | S/m |
| Sal(PSU) | Salinity | PSU |
| _R100_ | Scattering of 660 nm ligth at 100° | m-1 |
| _R125_ | Scattering of 660 nm ligth at 125° | m-1 |
| _R150_ | Scattering of 660 nm ligth at 150° | m-1 |
| CDOM-ppb | Fluorescence by colored dissolved organic matter. NOT CALIBRATED! | ppb |
| _G100_ | Scattering of 532 nm ligth at 100° | m-1 |
| _G125_ | Scattering of 532 nm ligth at 125° | m-1 |
| _G150_ | Scattering of 532 nm ligth at 150° | m-1 |
| CHL(ug/l) | Chlorophyll-a concentration, from fluorescence measurements | µg/l |
| m-1 | Beam attenuation coefficient at 660 nm | m-1 |

- BOUM_radiometer_XX.xls

General info about the radiometric casts

| cast number | date | time (UTC) | lat | lon | wind (m/s) | solar zenith angle (°) |
| --- | --- | --- | --- | --- | --- | --- |
| 7 | 2008-06-29 | 13:48:01 | 33° 42.67 | 32°39.72 | 3.6 | 52 |
| 8 | 2008-07-04 | 11:20:29 | 34° 07.58 | 18° 27.43 | 7.7 | 13 |
| 9 | 2008-07-04 | 14:23:20 | 34° 08.27 | 18° 28.98 | 6.2 | 47 |
| 10 | 2008-07-05 | 11:17:58 | 34° 03.67 | 18° 33.33 | 6.2 | 13 |
| 11 | 2008-07-05 | 14:36:49 | 34° 02.44 | 18° 36.12 | 5.1 | 50 |
| 12 | 2008-07-06 | 11:11:05 | 33° 58.44 | 18° 32.68 | 3.1 | 12 |
| 13 | 2008-07-06 | 14:33:23 | 33° 57.38 | 18° 31.60 | 5.7 | 50 |
| 14 | 2008-07-14 | 16:05:53 | 39° 06.16 | 5° 16.24 | 8.2 | 58 |
| 15 | 2008-07-15 | 12:04:29 | 39° 07.07 | 5° 10.75 | 9.8 | 18 |
| 16 | 2008-07-15 | 15:37:34 | 39° 06.43 | 5° 10.10 | 6.7 | 52 |
| 17 | 2008-07-16 | 12:14:59 | 39° 14.06 | 5° 09.31 | 4.1 | 19 |
| 18 | 2008-07-16 | 15:38:38 | 39° 16.16 | 5° 10.32 | 2.6 | 52 |

Parameters

| Column title | Description | Unit |
| --- | --- | --- |
| date | date (UTC) | yyyy-mm-dd |
| time | time (UTC) | hh:mm:ss |
| lat | latitude | degrees north |
| lon | longitude | degrees east |
| solz | solar zenith angle | degrees |
| wind | wind speed | m/s |
| pres | pressure | dbar |
| incli_x | sensor inclination x | degrees |
| incli_y | sensor inclination y | degrees |
| wavelength | wavelength | nm |
| Es | downwelling irradiance in air | W/(m2 nm) |
| Ed | downwelling irradiance in water | W/(m2 nm) |
| Lu | upwelling radiance in water | W/(m2 nm sr) |

- ap.xls

The file contains spectra of the particulate absorption, ap, for the cycle casts on the long 	duration stations.

The first column contains the wavelengths in nm. Each of the following columns contain the 	spectrum from one sample, as indicated by the first row of the column. BOUM_042_09 means 	CTD number 042, bottle 09.

The other absorption-related files have the same structure.

Parameters (one parameter per file):

| Name | Description | Unit |
| --- | --- | --- |
| OD_fp | optical density measured by spectrophotometer, before pigment extraction |  |
| OD_fd | optical density measured by spectrophotometer, after pigment extraction |  |
| beta_fp | beta correction factor, before pigment extraction |  |
| beta_fd | beta correction factor, after pigment extraction |  |
| ap | particle absorption coefficient | m-1 |
| ad | de-pigmented particle absorption coefficient | m-1 |
| aphi | phytoplancton absorption coefficient | m-1 |
| ap_chl_spec | chlorophyll specific particle absorption coefficient | m-1 |
| aphi_chl_spec | chlorophyll specific phytoplancton absorption coefficient | m-1 |

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |

- Calibration of the diffuse attenuation coefficient:

-H. Loisel and A. Morel, « Light scattering and chlorophyll concentration in case 1 waters: A 	reexamination », Limnol. Oceanogr., 43(5), 847-858 (1998).

- Protocol for absorption sampling, measurements and analysis:

Mitchell et al, « Determination of spectral absorption coefficients of particles, dissolved 	material and phytoplankton for discrete water samples », in « Ocean optics protocols for 	satellite ocean color sensor validation, revision 4 Volume IV. Inherent optical properties: 	instruments, characterizations, field measurements and data analysis protocols » NASA/TM-	2003211621 edited by J. L. Mueller, G. S. Fargion and C. R. McClain. Nasa Goddard Space 	Flight Center, Greenbelt, Maryland (2003).

- Calculation of the beta correction factor for the absorption measurements:

-Allali et al, « Spatial variations in the chlorophyll-specific absorption coefficients of 	phytoplancton and photosynthetically active pigments in the equatorial Pacific». JGR Vol. 102, 	No. C6 (1997).

-Bricaud et al, «Natural variability of phytoplanktonic absorption in oceanic waters: Influence of 	the size structure of algal populations». JGR Vol. 109, C11010 (2004).