META_INFORMATION_PARAMETRES / PARAMETERS

(à remplir par le responsable du paramètre /    to be filled by the PI    )

| Date de création | November 7th, 2008 | created on |
| --- | --- | --- |
| Date: de dernière mise à jour | December 3rd, 2008 | last update |

| ### PARAMETRES CONCERNES | ### PARAMETERS |
| --- | --- |

| Mode d’échantillonnage | No parametre | nom parametre | Status : [non] valid |
| --- | --- | --- | --- |
| Sampling | parameter number | name of the parameter | Status : [not] valid |
| Continuous sampling above the surface | P(274) with two colomns for different coefficients (mpT upd) | Cstar_cp(532nm) | Valid, restricted |
| Continuous sampling above the surface |  | Cstar_cp(650nm) | Valid, restricted |

| ### PROJET ETUDE | ### PROJECT TITLE |
| --- | --- |

BOUM

| ### RESPONSABLE SCIENTIFIQUE | ### PRINCIPAL INVESTIGATOR |
| --- | --- |

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| Michael | Behrenfeld | +1-541-737-5289 | +1-541-737-3573 | mjb@science.oregonstate.edu |
| Giorgio | Dall’Olmo | +1-541-737-5274 | +1-541-737-3573 | giorgiod@science.oregonstate.edu |

| ### BREVE DESCRIPTION DU PROJET | ### BRIEF DESCRIPTION OF PROJECT |
| --- | --- |

| ### DESCRIPTION DES PARAMETRES | ### PARAMETERS DESCRIPTION |
| --- | --- |

# Ce qui a été mesuré et comment  (méthode analytique et/ou références )/

# What did you measure and how did you do it (include references for analytical methods)?

Measurements of particulate beam-attenuation coefficient were collected on water continuously pumped from about 9 m below the sea surface.  The transmissometers used were two single-wavelength WETLabs C-stars, one at 532 nm and the other at 650 nm.  To account for the dissolved signals, instrumental drifts, and biofouling, a custom-made automatic valve directed

the bulk seawater through a 0.2-um Cole Palmer nylon cartridge filter, for ten minutes every hour.  To compute particulate beam attenuation, the median values of the beam attenuation coefficients measured during each of the 0.2-um filtration times were linearly and piecewise interpolated between hours and subtracted from the bulk measurements.

# Stratégie d'échantillonnage /

# Sampling strategy

Data were recorded continuously at 6Hz and binned every minute.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données /

# Post-cruise data analysis/treatment required, and the time frame for this

-

# Estimations des erreurs, précision, sensibilité des données /

# Error estimates, precision and accuracy of the data

Reported uncertainties (cstar_cp_err) account for the standard deviation in bulk and 0.2um-filtered signals recorded within the 1-minute bins.  Error were assumed to be independent.  The nominal 6Hz sampling rate corresponded to a median number of measurements per minute-bin of 347.

| ### FICHIERS | ### FILES |
| --- | --- |

# Nom de fichier de données /

# File name

Continuous_data_Cstar_particle_attenuation_coefficients.xls

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données /

# Data file structure, units, label columns, …

There are 7 columns of data separated by commas.  Each column is labeled (first row), units are reported on row 2.

The following table provides an example:

| Time since 2008/6/17 23:59:59 | Latitude | Longitude | cstar_cp(532) | cstar_cp(650) | cstar_cp_err(532) | cstar_cp_err(650) |
| --- | --- | --- | --- | --- | --- | --- |
| (Decimal days) | (Decimal degrees North) | (Decimal degrees East) | (1/m) | (1/m) | (1/m) | (1/m) |
| 1.600834e+001 | 3.413702e+001 | 1.844400e+001 | 5.253713e-002 | 4.443038e-002 | 1.353593e-003 | 1.203516e-003 |
| 1.600904e+001 | 3.413705e+001 | 1.844401e+001 | 5.255091e-002 | 4.443038e-002 | 1.353606e-003 | 1.202367e-003 |
| 1.600973e+001 | 3.413708e+001 | 1.844402e+001 | 5.256469e-002 | 4.443038e-002 | 1.353618e-003 | 1.202367e-003 |

| ### RESULTATS PRELIMINAIRES | ### RESULTS |
| --- | --- |

| ### REFERENCES BIBLIOGRAPHIQUES | ### REFERENCES - PAPERS |
| --- | --- |