FICHE  META_INFORMATION_PARAMETRES

(à remplir par le responsable du paramètre)

#### ( A renseigner AVANT la mission )

### PARAMETRES CONCERNES

79 - Phyto cyto

### OPERATION & CAMPAGNE / OPERATION & CAMPAIN

Operation  PECHE

Campain   DYNAPROC 2

### PROJET  ETUDE / PROJECT TITLE

Testing the hypothesis of the sustained wave train :

Denis M., V. Martin, A. Momzikoff, G. Gondry, L. Stemmann, S. Demers, G. Gorsky, V. Andersen. (2003) Pulsed remineralisation in the northwestern Mediterranean Sea: an hypothesis. J. Mar. Syst. 39: 19-41.

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # DENIS Michel | # LMGEM (UMR 6117) | # 334 91829114 | # 334 91826548 | # denis@com.univ-mrs.fr |
|  | # 163 avenue de Luminy, Case 901, 13288 Marseille cedex 09 |  |  |  |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

Objectives :

- to determine the temporal variation of  the vertical distribution of  ultraphytoplankton (< 10 µm) in the upper 200 m and discriminate different populations, namely : Prochlorococcus, Synechococcus, picoeukaryotes and nanoeukaryotes

- to convert abundances in terms of carbon biomass

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Seawater will be collected with Niskin bottles mounted on a CTD-Rosette at the following depths : 5, 20,40,60,80,100,150,and 200 m. Samples will be prefiltered on 100 µm mesh size net. Cryovials of 5 cm3 will be filled with 4.5 cm3 seawater and the aliquot fixed with 0.5 cm3 paraformaldehyde solution (PFA, 20%) in order to reach a final concentration of 2%. (Trousselier et al. 1995). The fixed sample will be frozen in liquid nitrogen and kept at low temperature (-80°C or 77K) until analysis in the laboratory.

Prior to anlysis, samples will be thawed and analysed with a flow cytometer (Cytoron Absolute, Ortho Diagnostic Systems) equipped with a 15 mW air-cooled 488 nm argon laser, the sample and sheath flow rates being 1 and 100 mm3 s-1 respectively. Each cell will be characterized by 5 optical parameters: two diffraction parameters, forward and right angle scatter, three fluorescence parameters related to emissions in the red (> 620 nm), orange (565-592 nm) and green (515-530 nm) wavelength ranges. Just before analysis, fluorescent beads of 3.6 µm in diameter will be added in order to normalize the flow cytometer settings and to provide a reference for concentration calculations (Grégori, 2001). Data will be collected and stored in list-mode with the Immunocount software (ORTHO Diagnostic Systems). Cluster analyses will be run with the WINLIST software (VERITY Software House Inc.).The instrument-counting reproducibility is better than 3.7% (SD, n = 23) over a range of concentrations extending one order of magnitude.

# Stratégie d'échantillonnage / Sampling strategy

Sampling for analytical flow cytometry analysis was defined by the hypothesis testing priorities which lead to the above mentioned sampled depths.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Added value to results from analytical flow cytometry analysis will come from comparisons and correlations with respiratory activity, hydrological and geochemical parameters

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

Reproducibilitry on counting is better than 3%.

### FICHIERS / FILES

# Nom de fichier de données / file name

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

### RESULTATS PRELIMINAIRES  / RESULTS

- Résolution par cytométrie en flux de 4 sous-populations : Prochlorococcus, Synechococcus, picoeucaryotes, nanoeucaryotes

- grande variabilité d’abondance dans les premiers 70 m entre profils successifs

- au cours du cycle 4, les abondances intégrées suivent une tendance croissante, sauf Prochlorococcus et la position des pics d’abondance tend à s’enfoncer

- l’effet du coup de vent se traduit par une augmentation notable des abondances entre les jours juliens 285 et 288

- les picoeucaryotes représentent la plus forte biomasse en termes de carbone sur l’ensemble du cycle 4

### REFERENCES BIBLIOGRAPHIQUES

Tavernier C. (2005) Variation à court terme de la distribution verticale de l’ultraphytoplancton en Méditerranée (campagne DYNAPROC 2, 13 septembre-18 octobre 2004). Mastère recherche Biodiversité, Paléontologie, Océanologie Biologique, Université des Sciences et Techniques de Lille.