Size limitation / UVP model

The UVPs did observe and measure correctly the particles larger than the Min ESD listed below:

| ## UVP Model | Min ESD (mm) | First column to keep in his files |  |
| --- | --- | --- | --- |
| 2a | 0.203 | 10 |  |
| 2c | 0.089 | 6 |  |
| 3a | 0.483 | 14 |  |
| 3b | 0.089 | 6 |  |
| 4a | 0.056 | 4 |  |

aHISbMOY.TXT files column description

a : Date (YYYYMMDD_HHMMSS000_)

Data Value are :

- b=NB : number of particles per litre ranging beetween ESD min and ESD max

- b=BV : biovolume of particles ranging beetween ESD min and ESD max (ppm)

|  | Col. N° | ESD min (mm) | ESD max (mm) |
| --- | --- | --- | --- |
| Sample Depth (5m step) | 1 |  |  |
| Sample Depth (observed) | 2 |  |  |
| Number of Computed Images | 3 |  |  |
| Data Value | 4 | 0.052 | 0.066 |
| Data Value | 5 | 0.066 | 0.083 |
| …. | 6 | 0.083 | 0.105 |
|  | 7 | 0.105 | 0.132 |
|  | 8 | 0.132 | 0.166 |
|  | 9 | 0.166 | 0.209 |
|  | 10 | 0.209 | 0.264 |
|  | 11 | 0.264 | 0.332 |
|  | 12 | 0.332 | 0.419 |
|  | 13 | 0.419 | 0.527 |
|  | 14 | 0.527 | 0.664 |
|  | 15 | 0.664 | 0.837 |
|  | 16 | 0.837 | 1.055 |
|  | 17 | 1.055 | 1.329 |
|  | 18 | 1.329 | 1.674 |
|  | 19 | 1.674 | 2.110 |
|  | 20 | 2.110 | 2.658 |
|  | 21 | 2.658 | 3.349 |
|  | 22 | 3.349 | 4.219 |
|  | 23 | 4.219 | 5.316 |
|  | 24 | 5.316 | 6.697 |
|  | 25 | 6.697 | 8.438 |
|  | 26 | 8.438 | 10.631 |
|  | 27 | 10.631 | 13.395 |
|  | 28 | 13.395 | 16.876 |
|  | 29 | 16.876 | 21.262 |
|  | 30 | 21.262 | 26.789 |

aHISbMOYRED.TXT files column description

a : Date (YYYYMMDD_HHMMSS000_)

Data Value are :

- b=NB : number of particles per litre ranging beetween ESD min and ESD max

- b=BV : biovolume of particles ranging beetween ESD min and ESD max (ppm)

| # Classe N° | ## Col N° | ESD min (mm) | ESD max (mm) |
| --- | --- | --- | --- |
| Sample Depth (5m step) |  | 0.052 | 0.091 |
| Sample Depth (observed) |  | 0.091 | 0.160 |
| Number of Computed Images | 4 | 0.160 | 0.279 |
| Data Value | 5 | 0.279 | 0.488 |
| Data Value | 6 | 0.488 | 0.852 |
| ... | 7 | 0.852 | 1.488 |
|  | 8 | 1.488 | 2.600 |
|  | 9 | 2.600 | 4.543 |
|  | 10 | 4.543 | 7.937 |
|  | 11 | 7.937 | 13.867 |
|  | 12 | 13.867 | 24.228 |
|  | 13 | 24.228 | 42.329 |
|  | 14 | 42.329 | 73.955 |

- D*.ASC files column description

* : reference of the ASC file is found in the ENTETE.XLS file

| Parametre | ## Col N° |
| --- | --- |
| Scan # | 1 |
| Depth (db) | 2 |
| Temp (°C) | 3 |
| Cond (Siemens/m) | 4 |
| Turb (FTU) | 5 |
| Fluo (RU) | 6 |
| Depth (m) | 7 |
| Salinity (PSU) | 8 |
| Density (kg/m3) | 9 |
| Pot Temp | 10 |
| Nul | 11 |

Note :

NaN replaces wrong data or missing data