# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre) / (to be filled by the person in charge of the parameter)

### PARAMETRES CONCERNES / Parameters

Name of the Parameter : d15N and d18O-NO3

### OPERATION & CAMPAGNE & PIs / OPERATION & CAMPAIGN&PIs

Sampling method : Rosette

Station number-Cast number :  1,2,4,8,10,15,17,30,35,38,42,44,45,46,47,50,55,63,68,81,87,94,97,107,111,113,120,

Operation code :

### PROJET  ETUDE / PROJECT TITLE

Campaign  NAME : KEOPS2

Project Leader: Stéphane Blain (stephane.blain@obs-banyuls.fr)

Address :	Laboratoire d'Océanographie Microbienne (LOMIC)

UMR 7621 CNRS UPMC

avenue du Fontaulé

66650 Banyuls sur mer

Chief Scientist:  Bernard Queguiner (bernard.queguiner@univ-amu.fr)

Address :	Institut Méditerranéen d'Océanographie

Université d'Aix-Marseille

UMR CNRS 7294/UR IRD 235

Campus de Luminy, case 901

F-13288 Marseille Cedex 09

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Frank Dehairs | # VUB, ANCH Pleinlaan 2 B-1050 Brussels, Belgium | # +32 (0)2 629 1265 | # +32 (0)2 629 1811 | # fdehairs@vub.ac.be |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling:

We subsampled a fraction (10 ml) of the filtered seawater (0.2 µm) intended for nutrient analysis. The samples were kept frozen (-20°C) till analysis.

Analytical procedure: (briefly, could be a short recall to a published reference):

Denitrifier method using Pseudomonas aureofaciens bacteria which transform NO3 in N2O. N2O is measured for d15N and d18O isotopic composition using a custom-build gas bench on-line with an IRMS (Delta V, Thermo). All analysed quantities of NO3 were 20 nmole. Reference materials used were USGS -32, -34, -35; IAEA-N3.

Units: ‰ vs Air (d15N) or SMOW (d18O)

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : end of 2012

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

Repeatability : for d15N = 0.19‰ ; for d18O = 0.4‰

Accuracy (for reference material IAEA N3): for d15N : better than 0.1‰; for d18O, better than 0.2‰

### REFERENCES BIBLIOGRAPHIQUES

Sigman et al., 2001. A bacterial method for the nitrogen isotopic analysis of nitrate in seawater zand freshwater, Anal. Chem., 73, 4145-4153

Casciotti et al. 2002. Measurement of the oxygen isotopic composition of nitrate in seawater and freshwater using the denitrifier method, Anal. Chem., 74, 4905-4912.