# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre) / (to be filled by the person in charge of the parameter)

### PARAMETRES CONCERNES / Parameters

Name of the Parameter : Fugacity of CO2 in surface waters (oceanic fCO2)

### OPERATION & CAMPAGNE & PIs / OPERATION & CAMPAIGN&PIs

Sampling method : 	Underway

Operation code :

### PROJET  ETUDE / PROJECT TITLE

Campaign  NAME : KEOPS2

Projetc Leader: 	Stéphane Blain (stephane.blain@obs-banyuls.fr)

Address :		Laboratoire d'Océanographie Microbienne (LOMIC)

UMR 7621 CNRS UPMC

avenue du Fontaulé

66650 Banyuls sur mer

Chief Scientist:  	Bernard Queguiner (bernard.queguiner@univ-amu.fr)

Address :		Institut Méditerranéen d'Océanographie

Université d'Aix-Marseille

UMR CNRS 7294/UR IRD 235

Campus de Luminy, case 901

F-13288 Marseille Cedex 09

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / # phone number | # adresse mél / # email address |
| --- | --- | --- | --- |
| # Lo Monaco, C. | # LOCEAN, Paris | # +33(0)1.44.27.33.94 | # claire.lomonaco@locean-ipsl.upmc.fr |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Underway measurement of fCO2 was performed during most of the cruise (from 11-Oct to 21-Nov) by pumping surface waters at the front of the ship (~8m depth). A mean value of fCO2 was computed  every 5 minutes.

Analytical procedure : (briefly, could be a short recall to a published reference):

The technique for fCO2 measurements was described in details by Poisson et al. (1993), Metzl et al. (1995, 1999), Jabaud-Jan et al. (2004). This instrumentation was also part of the international at-sea intercomparison of fCO2 systems conducted in the late 1990’s (Körtzinger et al., 2000). In short, sea surface water was continuously equilibrated using a "thin film" type equilibrator thermostated with surface seawater. The CO2 in the dried gas was then measured with a non-dispersive infrared analyser (NDIR, Siemens Ultramat 5F until 27-Oct. replaced with Ultramat 6F for the rest of the cruise). Standard gases (270 ppm, 375 ppm, 480 ppm) were used for calibration.

Units: µatm

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : January 2013.

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

Based on the analysis of replicate measurements obtained over a few hours when the ship was on station, we estimated the reproducibility to about 1 µatm untill 27-Oct and 0.7 µatm for the rest of the cruise.

### REFERENCES BIBLIOGRAPHIQUES

Jabaud-Jan A., N. Metzl, C. Brunet, A. Poisson and B. Schauer, 2004. Variability of the Carbon Dioxide System in the Southern Indian Ocean (20°S-60°S): the impact of a warm anomaly in austral summer 1998. Global Biogeochemical Cycles, Vol. 18, No. 1, GB1042,10.1029/2002GB002017.

Körtzinger, A., L. Mintrop, D.W.R. Wallace, K.M. Johnson, C. Neill, et al., 2000. The international at-sea intercomparison of fCO2 systems during the R/V Meteor Cruise 36/1 in the North Atlantic Ocean. Marine Chemistry 2 (2–4), 171–192.

Metzl, N., A.Poisson, F. Louanchi, C. Brunet, B. Schauer, B. Brès, 1995. Spatio-temporal distributions of air-sea fluxes of CO2 in the Indian and Antarctic Oceans: a first step. Tellus, 47B, 56-69.

Metzl, N., B.Tilbrook and A. Poisson, 1999. The annual fCO2 cycle and the air-sea CO2 flux in the sub-Antarctic Ocean. Tellus, 51B, 4, 849-861.

Poisson, A., N. Metzl, C. Brunet, B. Schauer, B. Brès, D. Ruiz-Pino and F. Louanchi, 1993. Variability of sources and sinks of CO2 and in the western Indian and Southern Oceans during the year 1991. J. Geophys. Res. 98, C12, 22,759-22,778.