# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre) / (to be filled by the person in charge of the parameter)

### PARAMETRES CONCERNES / Parameters

Name of the Parameter : Dissolved Inorganic Carbon (DIC)

### OPERATION & CAMPAGNE & PIs / OPERATION & CAMPAIGN&PIs

Sampling method : 	Underway

Operation code :

### PROJET  ETUDE / PROJECT TITLE

Campaign  NAME : KEOPS2

Projetc Leader: 	Stéphane Blain (stephane.blain@obs-banyuls.fr)

Address :		Laboratoire d'Océanographie Microbienne (LOMIC)

UMR 7621 CNRS UPMC

avenue du Fontaulé

66650 Banyuls sur mer

Chief Scientist:  	Bernard Queguiner (bernard.queguiner@univ-amu.fr)

Address :		Institut Méditerranéen d'Océanographie

Université d'Aix-Marseille

UMR CNRS 7294/UR IRD 235

Campus de Luminy, case 901

F-13288 Marseille Cedex 09

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / # phone number | # adresse mél / # email address |
| --- | --- | --- | --- |
| # Lo Monaco, C. | # LOCEAN, Paris | # +33(0)1.44.27.33.94 | # claire.lomonaco@locean-ipsl.upmc.fr |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Surface seawater was collected regularly (every 4-8h) in 500 ml bottles for the simultaneous analysis of dissolved inorganic carbon and total alkalinity. The samples were either measured directly, or poisoned with 300 µl of saturated solution of HgCl2, stored in the dark, in a cold environment, and measured within a few days.

Analytical procedure : (briefly, could be a short recall to a published reference):

Measurements were performed onboard following a potentiometric titration method (Edmond, 1970), using an automated system with a closed cell described by Goyet et al. (1991). The equivalence point is determined using a non-linear regression method (D.O.E., 1994). Measurements were calibrated using Certified Referenced Materials (CRMs, Batch 111, provided by Dr. A. Dickson, SIO, University of California). At least two CMRs were measured twice a day.

Units: µmol/kg

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : January 2013.

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

The standard deviation (2) of 12 replicate measurements was 1.7 µmol/kg. Accuracy was 4 µmol/kg based on the analysis of CRMs.

### REFERENCES BIBLIOGRAPHIQUES

D.O.E., 1994. Handbook of methods for analysis of the various parameters of the carbon dioxide system in sea water; version 2, A.G. Dickson and C.Goyet, eds. ORNL/CDIAC-74.

Edmond J. M., 1970. High precision determination of titration of alkalinity and total CO2 of seawater by potentiometric titration, Deep-Sea Research, 17, 737-750.

Goyet C., C. Beauverger, C. Brunet and A. Poisson, 1991. Distribution of carbon dioxide partial pressure in surface waters of the southwest Indian Ocean, Tellus, 43B, 1-11.