# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre) / (to be filled by the person in charge of the parameter)

### PARAMETRES CONCERNES / Parameters

Name of the Parameter : Sea Surface Salinity (SSS)

### OPERATION & CAMPAGNE & PIs / OPERATION & CAMPAIGN&PIs

Sampling method : 	Underway

Operation code :

### PROJET  ETUDE / PROJECT TITLE

Campaign  NAME : KEOPS2

Projetc Leader: 	Stéphane Blain (stephane.blain@obs-banyuls.fr)

Address :		Laboratoire d'Océanographie Microbienne (LOMIC)

UMR 7621 CNRS UPMC

avenue du Fontaulé

66650 Banyuls sur mer

Chief Scientist:  	Bernard Queguiner (bernard.queguiner@univ-amu.fr)

Address :		Institut Méditerranéen d'Océanographie

Université d'Aix-Marseille

UMR CNRS 7294/UR IRD 235

Campus de Luminy, case 901

F-13288 Marseille Cedex 09

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / # phone number | # adresse mél / # email address |
| --- | --- | --- | --- |
| # Lo Monaco, C. | # LOCEAN, Paris | # +33(0)1.44.27.33.94 | # claire.lomonaco@locean-ipsl.upmc.fr |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Underway measurement of salinity was performed during most of the cruise (from 11-Oct to 21-Nov) by pumping surface waters at the front of the ship (~8m depth). A mean value of salinity was computed every 5 minutes. Surface water was also collected regularly (every 8 hours) in 150 ml salinity bottles provided by Ocean Scientific International Ltd

Analytical procedure : (briefly, could be a short recall to a published reference): Samples were measured onboard, at the end of the cruise, using a Guidline Portasal salinometer (provided by Y.H. Park). IAPSO standards were used (Ocean Scientific International Ltd). Continuous salinity measurements obtained from the thermosalinograph (SBE 45) were calibrated against the salinometer data (Guidline PortaSal).

Units: psu

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery : January 2013

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

The accuracy of salinity measurements from the is 0.005 for the thermosalinograph data and 0.003 for the salinometer data. Reproducibility of the continous measurements was estimated to be better than 0.001, based on the analysis of replicate measurements obtained over a few hours when the ship was on station.

### REFERENCES BIBLIOGRAPHIQUES