# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre) / (to be filled by the person in charge of the parameter)

### PARAMETRES CONCERNES / Parameters

Name of the Parameter :dissolved (d) dPb, dCd, dAl, dMn, dCo, dNi, dCu

### OPERATION & CAMPAGNE & PIs / OPERATION & CAMPAIGN&PIs

Sampling method :Trace metal clean rosette

Station number-Cast number :

A3-1

R-2

E-1

TEW-1

TEW-2

TEW-3

TEW-4

E-2

TEW-5

TEW-7

E-3

F-L

G-1

E-4E

A3-2

E-4W-2

E-5

Operation code : TMR_PROD

### PROJET  ETUDE / PROJECT TITLE

Campaign  NAME : KEOPS2

Projetc Leader: Stéphane Blain (stephane.blain@obs-banyuls.fr)

Address :	Laboratoire d'Océanographie Microbienne (LOMIC)

UMR 7621 CNRS UPMC

avenue du Fontaulé

66650 Banyuls sur mer

Chief Scientist:  Bernard Queguiner (bernard.queguiner@univ-amu.fr)

Address :	Institut Méditerranéen d'Océanographie

Université d'Aix-Marseille

UMR CNRS 7294/UR IRD 235

Campus de Luminy, case 901

F-13288 Marseille Cedex 09

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Andrew Bowie | # ACE CRC Private Bag 80 Hobart Tasmania 7001 | # +613 6226 2509 | # +613 6226 2440 | # Andrew.Bowie@utas.edu.au |
| # Fabien Queroue | # IMAS Private Bag 129 Hobart Tasmania 7001 | # +61458 748 706 | # +613 6226 2440 | # Fabien.Queroue@utas.edu.au |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: The trace metal rosette was used to collect trace metal clean water samples. These were filtered inline using cartridge filters into acid washed, subsample containers within a class 100 clean container.

# Analytical procedure : preconcentration using Nobias PF1 chelate and analysed on SF-ICPMS adapted from:

Milne, A., Landing, W., Bizimis, M. & Morton, P. 2010. Determination of Mn, Fe, Co, Ni, Cu, Zn, Cd and Pb in seawater using high resolution magnetic sector inductively coupled mass spectrometry (HR-ICP-MS). Analytica Chimica Acta, 665, 200-207.

Biller, D. V. & Bruland, K. W. 2012. Analysis of Mn, Fe, Co, Ni, Cu, Zn, Cd, and Pb in seawater using the Nobias-chelate PA1 resin and magnetic sector inductively coupled plasma mass spectrometry (ICP-MS). Marine Chemistry, 130-131, 12-20.

Units: nmol/L

# Post-cruise data analysis/treatment required, and the time frame for this

Data will be quality controlled and then entered into MS excel for analysis. Timeframe – completed.

Estimated Date of Delivery :

January 2014

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

| Detection limit [pM] |  |
| --- | --- |
| Cd | 0.4 |
| Pb | 2.2 |
| Al | 1606.3 |
| Mn | 3.7 |
| Co | 2.8 |
| Ni | 2.6 |
| Cu | 107.8 |

|  | Safe S values (n=5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | certified [nmol/kg] |  |  | Measured [nmol/kg] |  |  |
| Cd | 0.001 | ± | 0.000 | 0.001 | ± | 0.001 |
| Pb | 0.047 | ± | 0.002 | 0.049 | ± | 0.002 |
| Al | 1.67 | ± | 0.100 | 2.504 | ± | 0.036 |
| Mn | 0.79 | ± | 0.060 | 0.887 | ± | 0.064 |
| Co | 0.005 | ± | 0.000 | 0.004 | ± | 0.001 |
| Ni | 2.31 | ± | 0.100 | 2.316 | ± | 0.103 |
| Cu | 0.51 | ± | 0.050 | 0.357 | ± | 0.020 |

|  | Geotraces Deep values (n=5) |  |  |  |
| --- | --- | --- | --- | --- |
|  | certified [nmol/kg] | Measured [nmol/kg] |  |  |
| Cd | 0.273±0.006 | 0.266 | ± | 0.013 |
| Pb | 0.042±0.001 | 0.043 | ± | 0.002 |
| Al | 17.7±0.1 | 18.315 | ± | 1.595 |
| Mn | 0.210±0.040 | 0.249 | ± | 0.008 |
| Co | 0.065±0.008 | 0.067 | ± | 0.005 |
| Ni | 3.960±0.140 | 3.978 | ± | 0.143 |
| Cu | 1.550±0.130 | 1.547 | ± | 0.078 |

### REFERENCES BIBLIOGRAPHIQUES

Milne, A., Landing, W., Bizimis, M. & Morton, P. 2010. Determination of Mn, Fe, Co, Ni, Cu, Zn, Cd and Pb in seawater using high resolution magnetic sector inductively coupled mass spectrometry (HR-ICP-MS). Analytica Chimica Acta, 665, 200-207.

Biller, D. V. & Bruland, K. W. 2012. Analysis of Mn, Fe, Co, Ni, Cu, Zn, Cd, and Pb in seawater using the Nobias-chelate PA1 resin and magnetic sector inductively coupled plasma mass spectrometry (ICP-MS). Marine Chemistry, 130-131, 12-20.