# FICHE  META_INFORMATION_PARAMETRES

# (à remplir par le responsable du paramètre) / (to be filled by the person in charge of the parameter)

### PARAMETRES CONCERNES / Parameters

Name of the Parameter : Ammonium

### OPERATION & CAMPAGNE & PIs / OPERATION & CAMPAIGN&PIs

Sampling method :

22 Niskin bottles-rosette

Station number-Cast number :

| Station number | Cast number |
| --- | --- |
| OISO | 002 |
| A3-1 | 004 |
| A3-2 | 103 ;107 ; 108 |
| TNS | 006 ; 007 ; 008 ; 009 ; 010 ; 011 ; 012 ; 013 ; 014 ; 015 |
| R | 016 ; 018 |
| E1-1 | 027 ; 028 ; 031 |
| E1-2 | 043 |
| E1-3 | 050 ; 051 ; 053 |
| E1-4W | 081 ; 082 ; 089 |
| E1-4E | 091 ; 093 ; 094 ; 095 |
| E1-5 | 114 ; 115 ; 117 |
| E4-W2 | 111 |
| TWE | 035 ; 037 ; 038 ; 042 ; 044 ; 045 ; 046 : 047 |
| NPL-L | 063 ; 064 ; 065 |
| NPF-S | 068 ; 069 |
| GEO-1 | 070 |
| IODA-RECOVERY | 120 |

Operation code : CTD

### PROJET  ETUDE / PROJECT TITLE

Campaign  NAME : KEOPS2

Project Leader: Stéphane Blain (stephane.blain@obs-banyuls.fr)

Address :	Laboratoire d'Océanographie Microbienne (LOMIC)

UMR 7621 CNRS UPMC

avenue du Fontaulé

66650 Banyuls sur mer

Chief Scientist:  Bernard Queguiner (bernard.queguiner@univ-amu.fr)

Address :	Institut Méditerranéen d'Océanographie

Université d'Aix-Marseille

UMR CNRS 7294/UR IRD 235

Campus de Luminy, case 901

F-13288 Marseille Cedex 09

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / address | téléphone / phone number | # fax / fax number | # adresse mél / email address |
| --- | --- | --- | --- | --- |
| Blain | Laboratoire arago, avenue du fontaulé, 66650 banyuls/mer | 33 (0) 468 887 334 | 33 (0) 468 887 395 | Stephane.blain@obs-banyuls.fr |

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Samples were collected from Niskin bottles in two 50 ml Schott glass bottles. Following rinsing, the bottles were filled with 40 ml of seawater and closed immediately to avoid contamination by air. Back in the inboard laboratory the OPA reagent was added. Samples were incubated for at least 3 hours in the dark, at ambient temperature before fluorescence measurements.

Reference :

Robert M. Holmes, Alain Aminot, Roger Kérouel, Bethanie A.Hooker, and Bruce J. Peterson : A simple and precise method for measuring ammonium in marine and fresh water ecosystems. 1999. Can. J. Fish. Aquat. Sci. vol 56, 1801-1808 .

Analytical procedure : (briefly, could be a short recall to a published reference):

The analysis is based on the reaction of ammonia with orthophtaldialdehyde and sulfite. The fluorescence is measured with an excitation of 360nm and an emission of 470nm with a Jasco FP-2020/2025 fluorometer.

Low nutrient seawater collected after the departure from La Reunion Island is used to prepare the standards. The concentration of the samples is calculated using the slope of the calibration curve. The background fluorescence of natural substances present in the sample is substracted from the sample-reagent fluorescence.

Units:  µmol/l

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Estimated Date of Delivery :  Delivered

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

Mean standard deviation is 10% for  concentrations > 0.1 µmol/l  and 20% for  concentrations < . 0.1 µmol/l

Detection limit : 0.006 µmol/l

Accuracy  :

Accuracy was determined by measuring certified standard CERTIPUR/Merck during the cruise. For a concentration of 5.4 µmol/l the accuracy was 5.9 %/

REFERENCES BIBLIOGRAPHIQUES

Robert M. Holmes, Alain Aminot, Roger Kérouel, Bethanie A. Hooker, and Bruce J. Peterson (1999)

A simple and precise method for measuring ammonium in marine and freshwater ecosystems. Can. J. Fish. Aquat. Sci. 56: 1801–1808

Brad W. Taylor, Christine F. Keep, Robert O. Hall, Jr., Benjamin J. Koch, Lusha M. Tronstad, Alexander S. Flecker and Amber J. Ulseth (2007)

Improving the fluorometric ammonium method: matrix effects, background fluorescence, and standard additions. J. N. Am. Benthol. Soc., 26(2):167–177