FICHE  META_INFORMATION_PARAMETRES

(à remplir par le responsable du paramètre)

### PARAMETRES CONCERNES / Parameters

Particulate P in aerosols

Particulate Ca in aerosols

Particulate Al in aerosols

Particulate Fe in aerosols

Particulate Mn in aerosols

Particulate Zn in aerosols

Particulate Co in aerosols

Particulate Cu in aerosols

Particulate Ni in aerosols

Particulate Cd in aerosols

Particulate 232Th in aerosols

Particulate Ti in aerosols

Particulate Sr in aerosols

Particulate Pb in aerosols

Particulate REE in aerosols

Parameter code: will be attributed later by the data base

### OPERATION & CAMPAGNE / OPERATION & CAMPAIN

Nom de l’opération (moyen de prélèvement/sampling tool): Aerosol Sampling System

Station number-Cast number:  see the compilation in the table below

Nom de la campagne/name of the cruise: PANDORA, (R/V Atalante, June 27th-Aug 7th, 2012)

### PROJET  ETUDE &PIs / PROJECT TITLE&PIs

THE PANDORA CRUISE, JULY 2012: AN INTEGRATED APPROACH OF THE CIRCULATION AND GEOCHEMISTRY IN THE SOLOMON SEA

PIs: Alexandre Ganachaud, Sophie Cravatte, Catherine Jeandel

Chief Scientist: Gérard Eldin (gerard.eldin@legos.obs-mip.fr)

All at LEGOS, Observatoire Midi-Pyrénées, Toulouse, Fr and IRD Center, Nouméa, NC

### RESPONSABLE SCIENTIFIQUE du paramètre / PI of the parameter

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Lacan | # LEGOS Toulouse | # 0561333043 |  | # francois.lacan@legos.obs-mip.fr |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

When flowing in the South-West Pacific and through the Solomon Sea, water masses undergo dynamical transformations as well as nutrients and micronutrients enrichments when entering in contact with the coasts of the Western Pacific. This impacts the productivity of the Equatorial Pacific Cold Tongue and therefore its climatic role. Trace elements that are injected from this “boundary processes” provide key information on mixing processes. Establishing the distribution of these elements and quantifying these land/ocean inputs is a priority of the international GEOTRACES program. Gaining a better knowledge of water mass transformations and pathways through the South-West Pacific is a priority of SPICE program (South Pacific Circulation and Climate Experiment).

In the frame of SPICE and GEOTRACES, the SOLWARA project focuses on these mechanisms in the Solomon Sea area, one of the key regions for the above mentioned surface and sub-surface water masses. This project includes historical physical and geochemical data analysis, deployment of autonomous observing platforms in the South-West Pacific, modelling, and the PANDORA oceanographic cruise with specific objectives and strategy. PANDORA is labelled as the GP12 GEOTRACES section (www.geotraces.org). The main objectives of the cruise are: to provide a quasi-synoptic description of surface and subsurface circulation in the Solomon Sea and in the straits connecting that sea with the equatorial circulation based on both hydrological and geochemical parameters; to deploy a series of moorings in the straits to obtain the temporal variability of the circulation; to evaluate water masses transformations and mixing; to document water/margins exchanges. The experimental approach will combine physical, chemical and geochemical experiments, which will give access to a wide range of space and time scales of the circulation and biogeochemical processes.

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été collecté, mesuré et comment / How was the parameter collected and measured (include references for analytical methods)?

Sampling: Aerosols were collected by pumping the air through ISOPORE 0.2µm pore size 47mm diameter polycarbonate membrane filters (Millipore; acid cleaned then dried within a laminar flow hood (INSU clean container)), at a flow rate of about 15 L/min. The system was protected from the rain. The device was set in front of the Bridge (E Deck). The pump was operated only when the apparent wind was lying within + or – 60° from the ship direction. The dual head pumping unit allowed measuring a blank systematically.

The sampling device was homemade (shipmade).

Filters were loaded and unloaded from the filter holders within a laminar flow hood (LEGOS Wooden Hood) and Teflon tweezers. After filtration the samples were placed in acid cleaned petri dishes.

Analytical procedure (briefly, could be a short link to a published reference):

To be filled up when then measurements will be done.

Synthèse échantillons collectés / Compilation of the collected samples (add a map of the cruise if possible)

TABLE

To be completed after the cruise.

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Data delivery estimated within 24 months from the cruise.

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

To be filled up when then measurements will be done

### FICHIERS / FILES (that will be sent to the data bas

# Nom de fichier de données / file name

To be filled up when then measurements will be done

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

To be filled up when then measurements will be done

### REFERENCES BIBLIOGRAPHIQUES