FICHE  META_INFORMATION_PARAMETRES

(à remplir par le responsable du paramètre)

#### ( A renseigner AVANT la mission )

### PARAMETRES CONCERNES

Major, trace elements and isotopes on suspended particles: LARGE VOLUMES FILTERED (Supor filter, >0.8 m, 147 mm)

### OPERATION & CAMPAGNE / OPERATION & CAMPAIN

Nom de l’opération: « In Situ Pump  (label ISP) »

Station number-Cast number :  see table below

Analysis of particulate (> 0.8 m) major and trace elements and some isotopes.

Considered elements and isotopes:

Particulate Bio-Si

Particulate 226Ra

Particulate 228Ra

Particulate 228Th

Particulate 234Th

Particulate 7Be

Particulate REE

Particulate Ba

Particulate P

Particulate Ca

Particulate Al

Particulate Mn

Particulate Zn

Particulate Co

Particulate Cu

Particulate Ni

Particulate Cd

Particulate Fe

Particulate Nd isotopes

Particulate Fe isotopes

Nom de la campagne: PANDORA, (R/V Atalante, June 27th-Aug 6th, 2012)

### PROJET  ETUDE / PROJECT TITLE

THE PANDORA CRUISE, JULY 2012: AN INTEGRATED APPROACH OF THE CIRCULATION AND GEOCHEMISTRY IN THE SOLOMON SEA

PIs: Alexandre Ganachaud, Sophie Cravatte, Catherine Jeandel

Chief Scientist: Gérard Eldin

### RESPONSABLE SCIENTIFIQUE / PRINCIPAL INVESTIGATOR

| # Nom / name | # adresse / # address | # téléphone / phone number | # fax / # fax number | # adresse mél / # email address |
| --- | --- | --- | --- | --- |
| # Catherine Jeandel | # GEOMAR, LEGOS, Toulouse | # +33.5.61.33.29.33 | # +33.5.61.25.32.05 | # Catherine.Jeandel@legos.obs-mip.fr |
| # FrancoisLacan | # GEOMAR, LEGOS, Toulouse | # +33.5.61.33.30.43 | # +33.5.61.25.32.05 | # Francois.lacan@legos.obs-mip.fr |

### BREVE DESCRIPTION DU PROJET / BRIEF DESCRIPTION OF PROJECT

When flowing in the South-West Pacific and through the Solomon Sea, water masses undergo dynamical transformations as well as nutrients and micronutrients enrichments when entering in contact with the coasts of the Western Pacific. This impacts the productivity of the Equatorial Pacific Cold Tongue and therefore its climatic role. Trace elements that are injected from this “boundary processes” provide key information on mixing processes. Establishing the distribution of these elements and quantifying these land/ocean inputs is a priority of the international GEOTRACES program. Gaining a better knowledge of water mass transformations and pathways through the South-West Pacific is a priority of SPICE program (South Pacific Circulation and Climate Experiment).

In the frame of SPICE and GEOTRACES, the SOLWARA project focuses on these mechanisms in the Solomon Sea area, one of the key regions for the above mentioned surface and sub-surface water masses. This project includes historical physical and geochemical data analysis, deployment of autonomous observing platforms in the South-West Pacific, modelling, and the PANDORA oceanographic cruise with specific objectives and strategy. PANDORA is labelled as the GP12 GEOTRACES section (www.geotraces.org). The main objectives of the cruise are: to provide a quasi-synoptic description of surface and subsurface circulation in the Solomon Sea and in the straits connecting that sea with the equatorial circulation based on both hydrological and geochemical parameters; to deploy a series of moorings in the straits to obtain the temporal variability of the circulation; to evaluate water masses transformations and mixing; to document water/margins exchanges. The experimental approach will combine physical, chemical and geochemical experiments, which will give access to a wide range of space and time scales of the circulation and biogeochemical processes.

#### ( A compléter APRES  la mission )

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

### DESCRIPTION DES PARAMETRES /  PARAMETERS DESCRIPTION

# Ce qui a été mesuré et comment / What did you measure and how did you do it (include references for analytical methods)?

Suspended particles were collected on 0,8 µm SUPOR membrane filters (147 mm diameters) using Large Volume filtration systems (In Situ Pumps, 5 McLane, 4 Challenger oceanic). Pumps were deployed on a new stainless steel cable and programmed for filtering for 4 hours, except at station PN077 where we had to shorten the collect down to 2 hours. A small vacuum filtration system allowed eliminating the remaining water on the filters before opening the pump’s heads. Then the filters were dried, aliquote was taken for bioSi and spectroscopic measurements and conditioned for low-level radioactivity counting. Sampled stations, depths and filtered volumes are reported in the following table.

# Stratégie d'échantillonnage / Sampling strategy

Seawater samples were taken at the depths of following stations:

# Décrire quels types de données sont nécessaires pour vous compléter votre propre jeu de données avant envoi à la base de données, et estimer le délai avant la disponibilité de vos données pour la base de données / Post-cruise data analysis/treatment required, and the time frame for this

Public access will be open after acceptance of the paper in preparation.

# Estimations des erreurs, précision, sensibilité des données / Error estimates, precision and accuracy of the data

### FICHIERS / FILES

# Nom de fichier de données / file name

The file “XXX” reports

# Explication des têtes de colonne, des unités et des abréviations utilisées dans le fichier de données / data file structure

### RESULTATS PRELIMINAIRES  / RESULTS

### REFERENCES BIBLIOGRAPHIQUES